Continuum Multiphysics Modeling for Electrochemical Systems in Python

Francisco Javier U. Galang1,2, and Justin C. Bui3*

1 Department of Chemical and Biomolecular Engineering

University of California Berkeley

Berkeley, CA, 94720, USA

2 Energy Technologies Area

Lawrence Berkeley National Laboratory

Berkeley, CA, 94720, USA

3Department of Chemical and Biomolecular Engineering

New York University, Tandon School of Engineering

Brooklyn, NY, 11238, USA

# Introduction to Continuum Multiphysics

Real electrochemical systems are composed of discrete atoms, ions, and molecules. Yet at the length scales relevant to real-device operation (e.g., micrometers to centimeters), individual molecular events average out into smooth, continuously varying fields: concentration, electric potential, temperature, velocity, etc. Continuum modeling exploits this averaging by replacing the discrete molecular picture with field variables governed by partial differential equations (PDEs). Rather than tracking every ion, atom, or molecule, continuum models follow field variables, such as concentration c(x,t), and solve for how they evolve in space and time.

Continuum approaches occupy the intermediate length and time scales between ab-initio molecular methods (quantum chemistry, molecular dynamics) and system-level approaches (technoeconomic models) (Figure 1). This mesoscale is where lab- and device-scale observables like polarization curves and concentration profiles live, making continuum models directly comparable to benchtop experiments and critical to the development and optimization of real electrochemical devices. Real devices at this scale are rarely governed by a single equation. Concentration, electric potential, temperature, and flow velocity each obey their own conservation law, but those laws are linked. For example, a reaction rate that depends on both local concentration and local potential couples the mass and charge conservation equations into a system that must be solved together. This sort of interdependence is what we refer to as “Multiphysics”. Solving these coupled equations in a continuum average is what constitutes a continuum multiphysics model.

Figure 1: Schematic depiction of continuum modeling in the context of the length and timescale of other modeling approaches.

Throughout this chapter, we provide a high-level introduction to the physics and numerical methods required to simulate electrochemical devices, then apply them to a concrete worked example in Python. We develop a continuum multiphysics model of the oxygen reduction reaction (ORR) at a porous PEMFC cathode. For context, a proton exchange membrane fuel cell (PEMFC) is an electrochemical device that converts the chemical energy of hydrogen directly into electricity by spatially separating H2 oxidation and O2 reduction: H₂ is split at the anode into protons and electrons, which travel to the cathode through the membrane and external circuit, respectively. There, they converge on Pt nanoparticles dispersed within a porous catalyst layer, driving ORR, a four-electron, four-proton reduction of O₂ to water.

This system is inherently multiphysics: the device simultaneously couples the physics of gas transport through the flow fields and porous media, dissolved O₂ and ion transport through ionomer binder in the catalyst layer, proton conduction through the membrane, and electron conduction through the carbon support, all linked through nonlinear electrochemical reaction kinetics that depend on the local concentration and potential of every phase at once.

# Continuum Modeling for Electrochemistry

## What can we learn?

Continuum models translate a set of governing physics, transport laws, kinetics, and conservation equations into quantitative predictions of measurable quantities. In electrochemistry and catalysis, the primary observables are polarization curves (current density vs. applied voltage) and faradaic efficiencies (fraction of current driving the reaction of interest). Validated continuum models (i.e., one that correctly predicts real-world behavior without overfitting) can then be interrogated for information that experiments cannot directly provide. For instance, they can resolve concentration and flux profiles at time and spatial resolutions that are not easily accessible to experimentalists, such as the local pH or cation concentration within a multi-phase porous medium.

## Why is it valuable?

The central advantage of continuum modeling is that it provides access to internal state variables (e.g., concentration, electrostatic potential, pressure, and temperature), that govern device behavior but are difficult to ascertain experimentally. A polarization curve collected by an experimentalist through voltammetry only provides a single observable (total current) at each applied voltage. A continuum model that reproduces these experimental variables enables deeper study into the complete state of the electrochemical device at any given position and time point to understand how its governing physics dictate its experimentally observed behavior. The examples below highlight data that continuum models make accessible, which are difficult to obtain experimentally yet carry significant diagnostic value for electrochemical devices.

- Current density breakdown: How does total current scale with applied voltage, and where does the polarization curve transition from kinetic to ohmic to mass-transport control? In the device, which charge-transport processes comprise the observed current?

- Voltage breakdown: What fraction of the applied overpotential is lost to reaction kinetics, ionic resistance, or electronic resistance, and what is the thermodynamic minimum applied voltage in the cell? This decomposition is only possible after solving the full field equations, and it directly identifies the limiting process at each operating voltage. This information would require multiple specialized experiments to infer indirectly.

- Faradaic efficiencies: For multi-product reactions, what fraction of total current drives formation of each product, and how does selectivity shift with current density and applied potential? Also, how does the faradaic efficiency vary with position along the length of a porous electrode or along-the-channel of a flow-field?

- Resolve chemical environments: What are the local concentrations of reactants, intermediates, and products inside the membrane, catalyst layer, and porous transport layer? How do these local environments dictate observed reaction kinetics in terms of activity and selectivity?

- Sensitivity analysis and optimization: How do efficiency metrics respond to changes in material properties or geometry, and what conditions maximize performance? Validated models enable sensitivity analysis and optimization: systematically varying material properties (e.g., diffusivity, conductivity, catalyst loading) or geometry (e.g., layer thickness) across a parameter space to identify the conditions that maximize performance, which guide experimental design and development. Continuum models can sweep parameters far more densely and cheaply than experiment allows. For instance, a membrane’s ion-exchange capacity can be numerically swept in a continuum model to identify a membrane that maximizes device performance. Experimentally changing this property would require synthesizing a new membrane for each tested ion-exchange capacity. Validated models can thus facilitate optimization and sensitivity campaigns with far greater speed and at lower cost.

Figure 2: Examples of data and insights that can be obtained via continuum modeling approaches that have utility in the development, optimization, and rationalization of electrochemical device performance and behavior.

The data-richness that becomes available upon solving and validating a continuum model goes beyond what traditional electroanalytical experiments can easily achieve. Reaching comparable fidelity often requires specialized setups or intensive beam-line experiments. This experimental complexity can be circumvented simply by solving the governing PDEs on a device-relevant mesh.

Continuum modeling is thus positioned at a scale that uniquely matches the scales of device-level experiments and measurements. Continuum models are coarser than molecular dynamics (e.g., these models do not directly calculate individual ion trajectories) but finer than system-level models that collapse the entire device into a lumped circuit element (e.g., those which treat the membrane as a resistor, or the double layer as a capacitor). The mesoscale in which continuum models lie (micrometers to millimeters and microseconds to steady state) matches the operative scale of most electrochemical devices. Models at this scale can therefore be validated against directly measured polarization data, providing real-world evidence of the validity of the modeling approach. The validated model can then be used to resolve physics in experimentally hard-to-reach or inaccessible domains.

# Continuum Modeling: Fundamentals and Numerical Methods

## Conservation Equations: Mass and Current

The governing partial differential equations solved in continuum modeling are conservation laws. For a scalar quantity ϕ (a concentration, a charge density, an energy) in a control volume, the statement is simple:

|  | ∂ϕ∂t=-∇⋅J+S | (1) |
| --- | --- | --- |

where J is the total flux of ϕ across the control volume boundaries and S is a volumetric source or sink. This single conservation equation generates the specific equations for each physical scalar quantity or field. The conservation law for species concentration is a second-order PDE in 1D

|  | ∂(c(x,t))∂t-ddxDeffdcdx=S(c,ϕs,ϕL) | (2) |
| --- | --- | --- |

In the governing equations that follow, this coupling appears explicitly through the source term S. When the source, S, in one balance depends on field variables governed by other balances (e.g., a reaction rate that depends on a concentration and/or on the local potentials, for instance) the equations can no longer be solved independently. They must instead be assembled into a single nonlinear system and solved simultaneously. Examples of specific source terms and coupled fields required for a PEMFC cathode model are introduced later in the chapter.

# What is a mesh?

To solve partial differential equations (PDEs) in conservation equations numerically, the continuous spatial domain is divided into a finite number of discrete cells or elements. This grid is what we refer to as a mesh. In a 1D finite-volume mesh, the domain 0,L is partitioned into N cells of width Δx=L/N (Figure 3b). Multiple domains with different properties can be stitched together to represent an electrochemical device. In 2D, the mesh is broken into control volumes, such as triangles (Figure 3c), and meshes of 2D dimensionality can be particularly important for capturing transport in two orthogonal directions that both control device behavior. For instance, flow runs along-the-channel (in-plane) of the electrolyzer, while transport proceeds through-plane across the components (Figure 3e). 3D meshes are discretized into control volumes, such as tetrahedra, and can be used to create faithful representations and simulations of an actual electrochemical device (Figure 3f). Panels (b), (d), and (f) of Figure 3 show these one-, two-, and three-dimensional mesh structures applied to a real electrochemical device — the PEMFC cathode that serves as this chapter’s worked example, developed in detail later when we build the full model.

Each cell has a centroid (i.e., its geometric center), where the field variable (concentration, potential) is stored and treated as the cell-averaged value. This stored value is the degree of freedom, or DOF, that the solver will determine.

Figure 3: Mesh discretization and PEMFC cathode model layout across spatial dimensions. The left column illustrates the generic finite-volume/finite-element mesh anatomy in one, two, and three dimensions; the right column shows the corresponding fuel-cell geometry. (a) One-dimensional cell-centered finite-volume mesh: scalar unknowns are stored at cell centers (nodes, ●), separated by cell faces (│); the shaded control volume i spans one cell of width Δx, and the boundary half-cell has width Δx/2. (b) Model layout for a fuel cell catalyst layer modeled in 1D: a gas-diffusion layer (GDL, N_GDL = 30 cells, 200 µm) and a more finely resolved catalyst layer (CL, N_CL = 50 cells, 10 µm) between the gas channel (x = 0) and the membrane (x = L). (c) Two-dimensional triangular mesh, showing a node, an edge, a single cell (element), and the characteristic element size h. (d) Two-dimensional analogue of the model domain, adding an open flow channel upstream of the GDL and CL, with boundary-layer refinement and the CL most finely meshed. (e) Three-dimensional tetrahedral mesh, with the node/edge/cell/element-size labels of (c) extended to 3D. (f) Three-dimensional cell geometry: a single continuous serpentine flow field on the membrane-electrode assembly, whose through-plane stack is expanded at right (cathode flow field C-FF, cathode gas-diffusion layer C-GDL, cathode catalyst layer C-CL, membrane MEM, and the mirrored anode layers (anode catalyst layer, anode gas-diffusion layer, anode flow field).

The governing PDE is then enforced as a discrete balance on each cell, converting the continuous PDE into a finite system of algebraic equations.

Between adjacent cells sit faces: the flat interfaces that separate one cell from its neighbor. Faces are not where variables are stored, but where fluxes are computed. A flux is simply the rate at which a quantity passes through a unit area of that face per unit time. For example, Fick's law states that the molar flux of O₂ across the face between cell i and cell i+1 is proportional to the concentration difference across it:

|  | Ji+1/2=-Deffci+1-ciΔx | (3) |
| --- | --- | --- |

The subscript i+1/2 denotes the face sitting halfway between centroids i and i+1. This finite difference approximates the true derivative dc/dx. The smaller Δx is, the more accurately it captures the local gradient and the lower the discretization error. However, halving Δx doubles the number of cells N, which increases both the memory required to store the system and the computational cost of solving it. In practice, one chooses N large enough that the solution no longer changes appreciably with further refinement (i.e., the solution is “mesh-independent”), balancing accuracy against cost. The governing PDE is then enforced as a discrete flux balance on each cell: whatever enters through the left face minus whatever leaves through the right face must equal the net source inside the cell. With a generic volumetric source S in cell i, this reads:

|  | Ji-1/2-Ji+1/2net flux in+SiΔxsource=0 | (4) |
| --- | --- | --- |

This is the discrete statement of ∇⋅J=S (the divergence theorem) where there is one algebraic equation per cell. Repeating this balance for all N cells converts the continuous PDE into a system of N equations in N unknowns, which can be solved by standard linear algebra or Newton iteration.

# From Mesh to Matrix

Once the domain is discretized into N cells, the physical state of the entire system is represented as a single solution vector u, where each entry corresponds to one field variable at one cell. Enforcing the discretized conservation equation on every cell produces one algebraic equation per DOF, collected into a residual vector F(u). The solver finds the u that satisfies:

|  | F(u)=0 | (5) |
| --- | --- | --- |

For a system with nvar field variables across N cells, the vectors are arranged in species blocks (all values of variable 1 across all cells, then all values of variable 2, and so on):

|  | u=ϕ1,1⋮ϕ1,Nϕ2,1⋮ϕ2,N⋮, Fu=F1,1⋮F1,NF2,1⋮F2,N⋮=0 | (6) |
| --- | --- | --- |

When the system is nonlinear, as for any electrochemical model with Tafel or Butler-Volmer kinetics, Fu=0 is solved iteratively by Newton-Raphson:

|  | uk+1=uk-J-1Fuk | (7) |
| --- | --- | --- |

Where J=∂F/∂u is the Jacobian, a matrix of partial derivatives of every residual equation with respect to every DOF. The solver iterates, recomputing J and F at each step, until Fu falls below a tolerance. In practice, Fu will never exactly reach 0, but gets close enough that the solution is physically meaningful. The criterion for what represents a “close-enough” value for the residual to zero is the convergence criterion for the solver.

By construction, each conservation equation in a cell depends only on its immediate neighbors. As a result, the Jacobian matrix is sparse and has a block-tridiagonal structure. One block per cell, with non-zero entries on the main block diagonal and the two adjacent block-diagonals. Prof. John Newman codified this structure into the BAND Algorithm (discussed in detail below), drastically cutting down the number of numerical operations needed to find a solution for an electrochemical system. However, as we move to higher dimensions, this structure is lost. The Jacobian grows to an enormous size, and dedicated solvers become necessary.

# Types of PDEs and their Numerical Methods

The conservation equation takes on different mathematical characteristics depending on what physics it describes. PDEs are classified into three families, each requiring a different numerical treatment:

Elliptic PDEs arise at steady state when the field variable is unchanging over time ∂ϕ∂t=0. A classic example is Laplace’s equation

|  | d2Φdx2=0 | (8) |
| --- | --- | --- |

The steady-state potential and concentration profiles are solutions to these elliptic PDEs. Because no time variable is present, these are solved as boundary value problems (BVPs). The solution is determined entirely by the boundary conditions (BCs). Numerically, discretization converts the elliptic PDE into a system of nonlinear algebraic equations, solved by multivariable Newton-Raphson iteration.1

Parabolic PDEs retain the time derivative and describe diffusion-dominated transient processes. A classic example is the heat equation or Fick’s second law.

|  | ∂T(x,t)dt=κd2Tdx2,∂C(x,t)dt=Dd2Cdx2 | (9) |
| --- | --- | --- |

In electrochemistry, transient models are of interest when we study the dynamics of a process and how a system evolves toward equilibrium, such as during a sudden potential step or while exploring battery-charging behavior. The standard numerical approach is the method of lines (MOL): discretize space first using the finite-volume method (FVM)2 or the finite-difference method (FDM)1 to convert the PDE into a large system of ODEs in time, much like we did for the elliptic PDEs, then integrate forward in time using explicit Euler or Runge-Kutta (scipy.integrate.solve_ivp in Python possesses a variety of other methods for this integration and is documented online). Parabolic problems require both boundary conditions and an initial condition.

Hyperbolic PDEs involve second-order derivatives in both time and space.

|  | ∂2ϕ∂t2-∂2ϕ∂x2=fx,t | (10) |
| --- | --- | --- |

Because this equation is second order in both time and space, the numerical model requires the initial time derivative of the field variable in addition to the initial condition and boundary conditions required for the elliptic and parabolic PDEs. Hyperbolic PDEs are uncommon in electrochemistry; they arise mainly in wave-like problems, whereas impedance methods such as EIS are usually treated as a frequency-domain linearization of the diffusion (parabolic) response. The standard numerical approach is again the method of lines, now splitting the equation into two coupled first-order PDEs. For example, let us say that a=ϕ, and b=∂ϕ∂t

|  | ∂a∂t=b, ∂b∂t=ϕi+1-2ϕi +ϕi-1Δx2+fx,t | (11) |
| --- | --- | --- |

After spatial discretization, this becomes a system of first-order ODEs in time that can be integrated forward using the same method-of-lines approach used for parabolic problems but now applied to a 2N dimensional state vector a,b. This approach of reducing higher order differential equation through multiple-first order differential equations is general.1

# Boundary and Initial Conditions for Electrochemical Systems

As alluded to previously, the number and type of conditions required to close a PDE system depend on its classification.

For elliptic (steady-state) problems, two boundary conditions per variable are required, one at each end of the domain. No initial condition is needed because there is no time-dependence.

For parabolic (time-dependent) problems, boundary conditions are still required at each spatial boundary, but an initial condition is also needed. The initial condition is the full spatial profile of every field variable at t=0 and in electrochemical systems, the natural initial condition is the equilibrium state. For example, by setting the concentration to a “bulk” value at all positions, potential to OCV or the equilibrium potential at all positions, and assume no current flowing:

|  | c(x,0)=cbulk,ϕ(x,0)=ϕOCV,t=0 | (12) |
| --- | --- | --- |

This represents the system before any perturbation is applied.

Even for steady-state (elliptic) problems where no formal initial condition exists, Newton-Raphson iteration requires an initial guess as the starting point for the iterative solver. The same physical intuition applies. Starting from the equilibrium state (uniform concentrations, OCV or equilibrium potential, and no current flowing) gives the solver the best chance of converging to the physical solution. A poor initial guess can cause convergence issues.

The three BC types that commonly appear in electrochemical systems are:

Dirichlet (fixed value): The variable is prescribed directly at the boundary. Examples: a large well-stirred reservoir maintains a fixed bulk concentration; a potentiostat holds the electrode at a specified potential ϕs=Vapplied.

Neumann (fixed flux): The spatial gradient of the field variable, through a flux, is prescribed. A zero-flux (insulating) Neumann BC, ∂ϕ/∂x=0, represents an impermeable wall or an electronically insulating membrane. The no-flux condition on ϕs at the CL/membrane interface is used in the example model.

Robin (mixed): The flux is coupled to the local value of the variable through an interfacial process. Consider the active catalyst surface where an electrochemical reaction consumes species at a rate set by Butler–Volmer kinetics:

-Deff∂c∂x∣surface=krxn(csurface,ϕsurface)
Because the flux (left side) depends on the gradient while the reaction rate (right side) depends on the surface value, this links both the value and the derivative.

Numerical Methods and Python Tools

Continuum models are implemented in one of two broad ways: by fully hard-coding the solvers, mesh, and iterative techniques using the core scientific-Python stack (NumPy for arrays and linear algebra, SciPy for solvers, and Matplotlib for visualization)3–5, or with specialized finite-element and finite-volume packages that supply meshing, assembly, and solvers out of the box. The hard-coded approach makes every step visible and interrogatable: the mesh generation, flux formula, residual, and Jacobian are all explicit, giving the user full control. It also carries no licensing cost and produces fully auditable code, lowering the barrier for open, reproducible research. This section surveys both routes and presents the canonical Newman BAND approach to one-dimensional electrochemical proble b ms. We also discuss what SciPy does and does not provide, and the higher-level packages available when a problem outgrows a hand-coded solver.

The Newman BAND Method: The Canonical Electrochemical Approach

Before modern scientific Python, the standard tool for 1D electrochemical continuum models was the BAND algorithm, introduced by John Newman in the 1960s and documented in Electrochemical Systems.1,6 The name BAND is short for “banded matrix” exploits the block-tridiagonal Jacobian structure described above to be solved in ON operations by Gaussian elimination rather than ON3. Newman's FORTRAN implementation of BAND became the de facto standard for decades of electrochemical simulation. Many published models in the fuel cell and battery literature were written directly in the BAND framework. Understanding BAND is still valuable because it makes the sparsity structure of the discretized equations explicit, but today's scientific Python stack provides more flexible alternatives with comparable performance for the problem sizes typical in 1D modeling.

Solving Finite Element Problems Using SciPy

Although SciPy does not contain FVM or FEM solvers, it provides the linear algebra backbone (scipy.linalg.solve and sparse solvers in scipy.sparse.linalg) that any discretization method ultimately requires. The discretization itself (converting PDEs into matrix equations) must be implemented by the user or by a specialized package.

The three main discretization families are summarized as follows:

| Method | Core idea | When to use | Python tools |
| --- | --- | --- | --- |
| FDM (finite difference) | Replace derivatives with difference quotients on a structured grid | Simple geometries, quick prototyping | NumPy (by hand), findiff |
| FVM (finite volume) | Integrate conservation law over each cell; fluxes are the primary unknowns | Conservation laws (mass, charge, momentum), a natural fit for electrochemistry | NumPy (by hand), FiPy |
| FEM (finite element) | Multiply PDE by test functions; minimize weighted residual | Complex geometries, structural mechanics, general PDEs | FEniCSx7, scikit-fem8, DOLFINx7 |

Specialized Packages

For problems beyond 1D uniform grids, several Python packages provide higher-level FVM/FEM infrastructure:

| Package | Method | Strengths | Typical use case |
| --- | --- | --- | --- |
| FiPy | FVM | Designed for diffusion-reaction-convection PDEs; syntax close to the continuous equations | Multi-component transport, phase-field models |
| FEniCSx / DOLFINx | FEM | Full PDE solver framework; handles 2D/3D unstructured meshes; automatic differentiation for Jacobian | Complex geometries, fluid mechanics |
| scikit-fem | FEM | Lightweight, NumPy-native; easy to customize | Teaching, small 2D/3D problems |
| OpenFOAM (via fluidfoam)9 | FVM | Industrial-grade computational fluid dynamics (CFD); handles turbulence, two-phase flow | Full 3D reactor modeling |
| COMSOL (via LiveLink) | FEM | Commercial; GUI-driven; broad physics library | Rapid prototyping, complex multiphysics |
| Firedrake10 | FEM | Open-source FEM via Unified Form Language (UFL) weak-form syntax; automatic C code generation; exact Jacobians by automatic differentiation; scalable parallel Portable, Extensible Toolkit for Scientific Computation (PETSc) solvers | Large-scale, parallel multiphysics on unstructured meshes |

The alternative is to these packages is to hand-code the discretization and is a useful teaching exercise. In this process, every operation is visible, traceable and interrogatable, including the mesh, the flux formula, the residual, and the Jacobian.

Example PEMFC Cathode ORR Model

To demonstrate the process of implementing a multiphysics continuum model for electrochemistry, we develop a one-dimensional model of the oxygen reduction reaction (ORR) at a porous PEMFC cathode. We first describe the physical system and the coupled fields it involves, then state the complete governing model and walk through its implementation in NumPy and SciPy. We then build the model in four stages, each switching on a single physical mechanism and validating it against a known limiting case before the next is added. The goal is not only to reproduce the cathode polarization curve but also to expose the internal concentration and potential fields that experiments cannot directly access.

Physical System and Model Schematic

The system is the cathode side of a proton-exchange membrane fuel cell (PEMFC) operating at 80 °C. A hydrogen fuel cell comprises seven primary components (Figure 4): an ion-exchange membrane that separates the anode and cathode reactions; an anode and cathode catalyst layer where the oxidation and reduction reactions occur, respectively; anode and cathode gas flow plates where H2 and O2 enter the device through convection; and porous anode and cathode gas-diffusion layers that ensure uniform delivery of H2 and O2 to the anode and cathode catalyst layers while also maintaining electronic conductivity.11,12 The anode and cathode catalyst layers are themselves complex porous media consisting of an ionomer binder that enables ionic transport to and from the catalyst particles, solid catalyst particles that host the electrochemical reactions, and carbon nanoparticles that enhance electronic conductivity. The void fraction in these catalyst layers facilitates the transport of the gaseous reactants from the porous transport layer to the catalyst particles. Because the ORR is kinetically limited compared to hydrogen oxidation, the phenomena occurring in the cathode largely dictate the performance of the overall fuel cell. Hence, modeling the ORR cathode provides a solid basis for understanding how the material properties and governing physics dictate the performance of the electrochemical device.

Molecular oxygen diffuses from a gas channel through the gas diffusion layer (GDL) and into the catalyst layer (CL), where it is reduced electrochemically to form H2O at Pt nanoparticles dispersed on a carbon support and coated with a proton-conducting ionomer.

The overall half-reaction is:

O2+4H++4e-→2H2O  U∘=1.229 V vs  Standard Hydrogen Electrode (SHE)

Figure 4: (Top) Schematic of a hydrogen fuel cell and inset of the key domain of the ORR catalyst layer and gas diffusion layer. (Bottom) 1-D continuum model representation of the ORR porous electrode domain.

The 1D spatial coordinate x runs from the GDL/CL interface (x=0) to the CL/membrane interface (x=LCL=10 μm). Three distinct phases coexist within the CL, each with its own volume fraction and transport properties. The values for the volume fraction were chosen to be representative of commercial catalyst layers:

| Phase | Volume fraction | Transport | Field variable |
| --- | --- | --- | --- |
| Ionomer (e.g. Nafion) | εI=0.43 | Dissolved O₂, ionic current | cO2,ion, ϕL |
| Solid (Pt/C) | εS=0.23 | Electronic current | ϕs |
| Gas pores | εG=0.34 | Gas-phase O₂ (Stage 3) | cO2gas |

The Complete Model

Before constructing the model in stages, we provide the key field variables and governing equations that dictate behavior of the complete, multiphysics system. These equations outline what physics are needed to model the desired through-plane voltage losses in the ORR cathode. The cathode is described by five coupled field variables, two source terms, and five conservation equations.

The five field variables are gas-phase O₂ in the GDL, gas-phase O₂ in the CL pores, the dissolved (ionomer) O₂ cO2,ion, the ionic potential ϕL, and the solid potential ϕs. N₂ is an inert background gas; it is fixed by closure (xN2 = 1 - xO2) rather than solved as an independent field.

Two source terms couple these fields. The reaction term is the volumetric ORR current density iORR=iORR(cO2,ion,ϕs,ϕL), set by Tafel kinetics and dependent on the dissolved O₂ and both potentials. The interphase term is the finite-rate phase transfer RPT=kv(KeqcO2,gas-cO2,ion), which carries O₂ from the pore gas into the ionomer.

The five conservation equations follow this source structure: the GDL gas-O₂ balance (diffusion only, no source); the CL pore-gas balance (source -RPT); the dissolved-O₂ balance (source +RPT-iORR/4F); the ionic-charge balance (source +iORR); and the electronic-charge balance (source +iORR).

Gas-phase transport is assumed to follow the binary Maxwell–Stefan stagnant-film flux N=-D∇c1-xO2, and all porous diffusivities and conductivities are corrected using Bruggeman effective medium theory. A Dirichlet condition fixes the gas-phase O₂ at the flow channel, a no-flux boundary for dissolved O₂ is applied to both faces, and the potentials carry the cell current at the contacts (the full set of Boundary Conditions appears below). The Field / Flux / Source table summarizes the four transported quantities (gas O₂ spans both the GDL and CL pores, so the four occupy five coupled fields) and their couplings:

| Field, ϕ | Flux, J | Source, S |
| --- | --- | --- |
| Dissolved (ionomer) O₂ cO2,ion | Fick's law -Deff∇c | Interphase gain minus ORR consumption, +RPT-iORR/4F (4 electrons transferred per O₂ molecule) |
| Gas-phase O₂ cO2,gas | Maxwell–Stefan -Dgas∇c1-xO2 (the (1-xO2) correction applies in both the GDL and CL pore gas) | Interphase transfer to ionomer -RPT (in the CL pores; none in the GDL) |
| Ionic (electrolyte) potential ϕL | Ohm's law iL=-κeff∇ϕL Am2 | -iORR Protons consumed at Pt Am3 |
| Solid (electronic) potential ϕs | Ohm's law iS=-σeff∇ϕs | iORR Electrons drawn from solid phase to drive reduction Am3 |

Governing Equations

Three coupled conservation equations govern the steady-state system, consisting of mass and charge conservation in the electrochemical system.

|  | ddxDeffdcO2dx=iORR4F | (13) |
| --- | --- | --- |
|  | ddxκeffdϕLdx=iORR | (14) |
|  | ddxσeffdϕsdx=-iORR | (15) |

Adding  (14) and (15) enforce charge conservation.

ddxiL+iS=0

The coupling term (present in all three equations) is the volumetric ORR current density iORRAm3of Cl, given by concentration-dependent Tafel kinetics:

|  | iORR=-diLdx=disdx=avi0cO2crefγexp-αcFηRT | (16) |
| --- | --- | --- |

where η=(ϕs-ϕL)-Ueq(cO2) is the local kinetic overpotential and av=2×107 m-1 is the volumetric Pt surface area. Equation 16 represents the Tafel limit of Butler-Volmer kinetics and is valid when the cathodic overpotential is large enough that the anodic current is negligible. In code:

# kinetics.py def R_ORR(ln_cO2, phi_s, phi_L, p): c_O2 = np.exp(ln_cO2) U_eq = p.U_ORR_eq(c_O2) # Nernst-corrected equilibrium eta = (phi_s - phi_L) - U_eq # local overpotential conc = np.maximum(c_O2 / p.c_O2_ref, 0.0) ** p.gamma arg = np.clip(-p.alpha * p.F * eta / (p.R*p.T), -700., 700.) return p.a_v * p.i0 * conc * np.exp(arg) # [A/m³_CL]

The argument clamp (np.clip) prevents the exponential from overflowing at large overpotentials.

The O₂ concentration is log-stored, meaning that we solve for ln(cO2) rather than cO2 itself. This choice has two benefits: it enforces positivity automatically (since c=eu>0 for any finite u), and it keeps cO2 well-scaled even as it depletes by several orders of magnitude across the catalyst layer under mass-transport limitation. The dissolved-O₂ concentration is about 10⁻¹ mol/m³ at the inlet; what motivates log-storage is its wide dynamic range, not its absolute value. A separate mismatch between concentration and potential scales, cO2 ∼ 10⁻¹ mol/m³ versus φ ∼ 1 V, is addressed by row scaling below.

Boundary Conditions and Initial Values

Each second-order ODE requires two boundary conditions. The full set is:

| Variable | x=0 (GDL/CL) | x=LCL (CL/membrane) |
| --- | --- | --- |
| cO2 | Dirichlet: c=cbc (Henry's law) | No-flux: dc/dx=0 (Stage 1/2) |
| ϕL | No-flux: dϕL/dx=0 (no ionic current crosses into GDL) | Dirichlet: ϕL=0 V (membrane reference) |
| ϕs | Dirichlet: ϕs=Vcathode (applied potential) | No-flux: dϕs/dx=0 (no electron current into membrane) |

Dirichlet at x=0, ϕs=Vcathode: The carbon support is connected to the external circuit. The applied cathode potential is held fixed here; it is the control variable swept during the simulation.

No-flux (Neumann) at x=LCL, ϕs: No electronic current flows into the proton-conducting membrane. The solid-phase current has been entirely converted to ionic current by the time it reaches the membrane.

Dirichlet at x=LCL, ϕL=0: The membrane surface is taken as the electrostatic reference potential for the ionic phase. All ionic potential values inside the CL are measured relative to this.

No-flux at x=0, ϕL: Ionic current (protons) cannot cross into the electronically conducting GDL; it enters the CL from the membrane side only.

The physical interpretation of these boundary conditions is that solid current enters from the GDL side and is gradually converted to ionic current across the CL by the distributed ORR. At the GDL face, all current is electronic; at the membrane face, all current is ionic. This current handoff is a fundamental feature of porous electrode theory, first formalized by Newman and Tobias.13

For the initial conditions in the system, we use the values of all variables at equilibrium when the cell is at open circuit.

# solver.py — equilibrium initial guess ln_cO2 = np.full(N, np.log(p.c_O2_bc)) # uniform O2 at inlet value phi_L = np.zeros(N) # no ionic potential drop phi_s = np.full(N, V_cathode) # uniform solid potential = applied voltage

This is not an arbitrary choice. At open circuit (with no current flowing) these are the exact solutions to all three conservation equations, making them a natural and stable starting point.

Multiphysics Coupling and the Degree of Freedom Vector

The three equations cannot be solved independently because iORR depends on all three unknowns simultaneously. We therefore assemble them into a single nonlinear system F(u)=0 where the DOF vector u concatenates all field values across all cells:

u[0·N : 1·N] = ln(c_O2) # log-stored dissolved O2 u[1·N : 2·N] = phi_L # ionic potential u[2·N : 3·N] = phi_s # solid potential

The finite volume (FV) residual for each cell assembles the three conservation equations. For cell i:

# assembly_stage1.py — FV residual assembly i_ORR = R_ORR(ln_cO2, phi_s, phi_L, p) # coupling term J_O2 = diffusion_face_fluxes(ln_cO2, dx, p.D_O2_eff, bc_left=p.c_O2_bc, bc_right=None) i_L = ohmic_face_fluxes(phi_L, dx, p.kappa_L_eff, bc_left=None, bc_right=p.phi_L_mem) i_s = ohmic_face_fluxes(phi_s, dx, p.sigma_s_eff, bc_left=V_cathode, bc_right=None) R_O2 = J_O2[:-1] - J_O2[1:] + (-i_ORR / (N_ELEC * p.F)) * dx R_phiL = i_L[:-1] - i_L[1:] + (+i_ORR) * dx R_phiS = i_s[:-1] - i_s[1:] + (-i_ORR) * dx

Each residual has the form Fi=Ji-1/2-Ji+1/2+SiΔx: net flux out of the cell plus source equals zero.

Newton Solver and Voltage Continuation

As mentioned previously, the assembled system F(u)=0 is nonlinear due to the exponential in iORR. We solve it with a Newton method with step clamping. In other words, given a current iterate u(k), compute the Jacobian J=∂F/∂u, then update:

u(k+1)=u(k)-J-1F(u(k)). The Jacobian is computed by forward finite differences, perturbing each DOF by h=ε⋅max(|uj|,1) with ε=10-7 and measuring the change in residual. This requires NDOF residual evaluations per Newton step but needs no analytical derivatives. For further discussion of numerical methods, we recommend consulting Numerical Methods for Engineers, S. C. Chapra and R. P. Canale, McGraw Hill.14

Two practical issues arise:

Row scaling. The O₂ residuals (scale ∼10-9) and potential residuals (scale ∼108) differ by many orders of magnitude. This is due to the natural scales at which we measure or report these variables. Without scaling, the linear solver treats the small residuals as negligible. The fix is to normalize each equation by its own infinity norm before solving:

scale = np.maximum(np.abs(J).max(axis=1), 1e-30) J_sc = J / scale[:, None] F_sc = F0 / scale du = scipy.linalg.solve(J_sc, -F_sc)

Step clamping. Near the starting guess, Newton steps can be large enough to push the iterate into unphysical territory (negative concentrations, potential jumps of volts). We bound each Newton step, clipping the update ∂u before it is applied (at most 5 log-units in lncO2 and 0.2 V in each potential).

# solver.py du[0*N:1*N] = np.clip(du[0*N:1*N], -5.0, 5.0) # max 5 log-units in ln(c_O2) du[1*N:2*N] = np.clip(du[1*N:2*N], -0.2, 0.2) # max 200 mV in phi_L du[2*N:3*N] = np.clip(du[2*N:3*N], -0.2, 0.2) # max 200 mV in phi_s

To generate a full polarization curve, we start at a voltage close to OCV (1.00 V) where the solution is nearly trivial, solve Newton to convergence, then use that converged solution as the initial guess for the next voltage step. The step size adapts based on convergence difficulty:

# solver.py — adaptive step size logic if n_iter <= 5: dV = min(dV * 1.5, dV_max) # easy convergence: grow step elif n_iter > 20: dV = max(dV * 0.5, dV_min) # slow convergence: shrink step # if Newton fails entirely: halve dV and retry same voltage

This produces a dense, automatically resolved polarization curve from OCV down to the mass-transport-limited regime at 0.35 V with no user tuning.

Model Implementation

Having specified the complete model, we now turn to its implementation. The model is built as a small, transparent NumPy and SciPy codebase in which every step of the discretization and solve is clear. We walk through the driver script, the parameter and mesh objects, and the modules that assemble and solve the discrete system, before building the model in four validated stages.

We begin by showing the main script (run_stage1.py). All models in this project share a common setup pattern. This project requires three external packages: NumPy (arrays and linear algebra generation), SciPy (dense linear algebra solver via scipy.linalg.solve), and Matplotlib (plotting). The driver script (run_stage1.py) imports from each module, instantiates the two core objects (p = Params() and mesh = make_mesh(p)), and passes them to specific functions. Every function in the project receives either p, mesh, or both as arguments. Understanding what these two objects hold is sufficient to read any subsequent code snippet.

# run_stage1.py — master setup from params import Params from mesh import make_mesh from kinetics import R_ORR from transport import diffusion_face_fluxes, ohmic_face_fluxes from assembly_stage1 import residual_stage1, compute_current from solver import voltage_sweep from plot_results import plot_polarization, plot_profiles p = Params() # single authoritative parameter object — passed everywhere as "p" mesh = make_mesh(p) # reads p.L_CL and p.N_CL; returns mesh.dx, mesh.xc, mesh.xf

Organizing Parameters: A Dataclass as a Physics Container

A recurring practical challenge in multiphysics modeling is managing the large number of physical constants, geometry parameters, and derived quantities. It is good practice to use a Python dataclass to build a self-consistent parameter container. This params.py class contains all physical constants, geometry, and material properties. Derived quantities (effective transport coefficients, boundary condition values) are computed automatically on instantiation via __post_init__ and refreshed by calling p.recompute() after any change.

# params.py @dataclass class Params: # Physical constants F: float = 96485.332 # C/mol (Faraday) R: float = 8.314463 # J/mol/K T: float = 353.15 # K (80°C) # Geometry L_CL: float = 10e-6 # m (catalyst layer thickness) N_CL: int = 50 # number of FV cells # Transport and kinetic parameters (...) # Derived quantities — auto-computed, do not set manually D_O2_eff: float = field(init=False) # Bruggeman-corrected O2 diffusivity kappa_L_eff: float = field(init=False) # effective ionic conductivity sigma_s_eff: float = field(init=False) # effective electronic conductivity (...) def __post_init__(self): self.recompute() def recompute(self): self.D_O2_eff = (self.eps_I ** self.brugg) * self.D_O2_water * self.ionomer_factor (...)

Derived quantities like the Bruggeman-corrected effective diffusivity Deff=εI1.5⋅Dbulk are recomputed automatically when Params is instantiated, and can be refreshed after any parameter change by calling p.recompute().

Building a Mesh in NumPy

The first step in any continuum simulation is constructing the mesh. For a 1D cell-centered finite-volume mesh, this is accomplished by dividing the domain 0,L into N uniform cells of width Δx = L/N, place a degree of freedom at each cell centroid, and tracking the face positions where fluxes are evaluated.

In this project, the mesh is generated in a Python dataclass, mesh.py. It holds the geometry of the 1D grid, built from p via make_mesh(p), which reads p.L_CL and p.N_CL. All flux calculations downstream use mesh.dx as the cell width.

# mesh.py @dataclass class CLMesh: L: float # domain length [m] N: int # number of cells def __post_init__(self): self.dx = self.L / self.N # cell width self.xc = (np.arange(self.N) + 0.5) * self.dx # cell centers (DOF locations) self.xf = np.arange(self.N + 1) * self.dx # face positions (flux locations) def make_mesh(p) -> CLMesh: return CLMesh(L=p.L_CL, N=p.N_CL) def make_gdl_mesh(p) -> CLMesh: return CLMesh(L=p.L_GDL, N=p.N_GDL)

xc holds the N cell-center coordinates where field variables (cO2, ϕL, ϕs) are stored. xf holds the N+1 face positions where fluxes are computed. The offset of 0.5 in the centroid formula is the defining feature of a cell-centered scheme. The first cell center sits half a cell-width in from the boundary, not on it.

For multi-domain problems (Stage 3), separate meshes are created for each domain. In this example, the GDL and CL have their own separate meshes, and their DOF vectors are concatenated. This is discussed further in Section 4.

cl_mesh = make_mesh(p) # 50 cells over L_CL = 10 μm gdl_mesh = make_gdl_mesh(p) # 30 cells over L_GDL = 200 μm

No external meshing library is needed for 1D uniform grids. For 2D/3D or unstructured problems, dedicated tools such as Gmsh or meshio can be used. The remaining modules each serve a different function for the model:

| File | Content |
| --- | --- |
| kinetics.py | Volumetric ORR rate [A/m³] R_ORR(ln_cO2, phi_s, phi_L, p) |
| transport.py | Fick/Ohm fluxes at cell faces diffusion_face_fluxes(...), ohmic_face_fluxes(...) |
| assembly_stage1.py | Assembles the full FV residual vector F(u) residual_stage1(u, mesh, p, V) |
| solver.py | Newton iteration and voltage sweep newton_solve(...), voltage_sweep(...) |
| cache.py | Binary cache for voltage sweep results save_cache(...), load_cache(...) |
| plot_results.py | Diagnostic figures plot_polarization(...), plot_profiles(...), etc. |

This project uses the FVM implemented entirely in NumPy. The face-flux functions in transport.py are the core of the discretization:

# transport.py — Fick's law face fluxes (FVM) def diffusion_face_fluxes(ln_cO2, dx, D_eff, bc_left, bc_right): c = np.exp(ln_cO2) # recover concentration from log-stored DOF J = np.empty(len(c) + 1) J[0] = -D_eff * (c[0] - bc_left) / (0.5 * dx) # left Dirichlet BC J[1:-1] = -D_eff * np.diff(c) / dx # interior faces J[-1] = 0.0 # right no-flux BC return J # shape (N+1,1)

Called in the assembly as:

# assembly_stage1.py J_O2 = diffusion_face_fluxes(ln_cO2, mesh.dx, p.D_O2_eff, bc_left=p.c_O2_bc, bc_right=None)

The FV residual for the O₂ conservation equation at cell i is then simply:

R_O2 = J_O2[:-1] - J_O2[1:] + (-i_ORR / (4*F)) * dx

This is the discrete statement of ∇⋅J=S: flux in minus flux out plus source equals zero.

Building and Validating the Model in Four Stages

Once the meshes are constructed, the model is solved in four stages where new physics are added at each new stage and coupled to the physics of previous stages. At every stage, the model builds in complexity and accuracy to approach the physical system. Staged construction is how many complex multiphysics models are developed and debugged. Each stage adds one to a few, simple physical mechanisms, introducing at most one new numerical technique, and is validated by confirming that it reduces to a known limit before the next stage is added. The map below summarizes the four stages; each is then treated as a self-contained lesson with the same three-part shape (Mechanism, Method, and Limit-check), closing with its Result.

| Stage | Mechanism switched on | New numerical method/addition | Validated by (limit) |
| --- | --- | --- | --- |
| 1 | Dissolved-O₂ diffusion + Tafel ORR + ohmic charge (the CL core) | Full finite-volume residual + Newton with voltage continuation | Three-way current consistency |
| 2 | Finite membrane O₂ permeability | Robin / flux boundary condition | kMT≈0 identical to Stage 1 |
| 3 | Gas-phase O₂ in the GDL and CL pores (dissolved O₂ by Henry equilibrium) | Multi-domain mesh concatenation; harmonic-mean interface diffusivity | D_gas ≈ 54,000× D_ion ⇒ realistic limiting current |
| 4 | Finite-rate interphase transfer R_PT + Maxwell–Stefan gas flux (complete model) | Extra DOF block (NG + 4·NC); (1 − x_ O₂) Stefan factor | kv → ∞ ⇒ Stage 3 ctot → ∞ ⇒ Fick |

Figure 5: Full implementation scheme of the 1D model for the ORR cathode in a PEMFC.

Stage 1: Minimal core (CL only)

Mechanism: Ionomer-phase dissolved-O₂ diffusion, Tafel ORR kinetics, and ohmic charge conservation (ϕL and ϕs) in the catalyst layer only, with a zero-flux O₂ condition at the impermeable membrane. This is the simplest physically complete model that still produces a polarization curve.

Method: The full numerical machinery is introduced here, once, on the simplest system: finite-volume residual assembly, log-stored concentrations, a finite-difference Jacobian, row scaling, per-DOF step clamping, and Newton iteration under natural voltage continuation. (The solver code listed under “Newton Solver and Voltage Continuation” above belongs to this stage.)

Limit-check: Three-way current consistency. The volumetric ORR integral equals the solid-phase flux at the catalyst-membrane interface. This must also equal the ionic flux at the membrane agreeing to within 1%. Additionally, we have implemented additional physicality checks (ϕs monotone, ϕL ≥ 0, near-OCV current ≈ 0) that the model passes confirming the physics are being solved correctly.

Result. A kinetically limited polarization curve in which ohmic losses are negligible. The ionomer is about 7× more resistive than the carbon, but both IR drops remain tiny at Stage 1 currents.

Figure 6: Stage 1 polarization curve (Vcathode vs. J). The steep near-OCV drop is governed by Tafel kinetics; the mild mass-transport tail reflects ionomer-phase O2 depletion near the membrane.

The Stage 1 polarization curve (Figure 6) shows two characteristic regimes: a steep near-OCV drop dominated by Tafel kinetics, and a gradual mass-transport tail at higher current densities as O₂ is depleted near the membrane. The spatial profiles in Figure 7 explain that tail, because at high absolute overpotentials (dark curves, low V) the reaction localizes near the GDL/CL interface, where O₂ is consumed before it can penetrate deep into the CL.

Figure 7: Stage 1 spatial profiles at four sampled voltages. (a) O2 concentration; (b) ionic potential ϕL (mV); (c) solid potential ϕs; (d) kinetic overpotential η=ϕs-ϕL-Ueq(mV). Dark curves correspond to higher overpotential; at high overpotential the reaction is concentrated near the GDL/CL interface (x = 0).

Stage 1 currents do not exceed about 3 mA/cm², at which the ionic and solid-phase ohmic losses together stay below 0.1% of the total overpotential, so no ohmic regime is discernible at this scale. Figure 8 makes this explicit using an applied voltage breakdown analysis.15 The kinetic overpotential accounts for more than 99% of the voltage loss, and on the expanded millivolt scale the ionomer is the more resistive phase (its Bruggeman-corrected ionic conductivity, ≈1.6 S/m, is roughly 7× lower than that of carbon ≈11 S/m), so the ionic IR drop dominates the otherwise-negligible ohmic loss. There is an additional mass transport loss as there is currently no gas phase to replenish the depleting O2.

Figure 8: Voltage-loss breakdown for Stage 1. Left: stacked-area decomposition showing kinetic overpotential accounting for >90% of total loss. Right: ohmic losses and mass transport loss on an expanded millivolt scale. The proton-conducting ionomer is the more resistive phase (its conductivity is roughly 7× lower than that of carbon, so the ionic IR drop dominates over the solid IR drop). Though, both ionic and solid ohmic drops are negligible at Stage 1 currents. As there is no gas phase to replenish the depleting O2, mass transport loss dominates.

Finally, Figure 9 shows the complementary face-centered currents: the solid-phase current is falls and the ionic-phase current iL rises across the CL.

Figure 9: Face-centered flux profiles at sampled voltages. Left: O₂ diffusion flux J_O2(x) entering at the GDL face (x = 0) and falling to zero at the no-flux membrane face (x = L_CL). Right: solid-phase is (solid lines) and ionic-phase iL (dashed lines) current densities.

Stage 2: Membrane O₂ boundary

Mechanism: Finite O₂ permeability of the membrane, introduced as a Robin boundary condition JO2=-kMTcO2 at the membrane face, replacing the Stage 1 zero-flux condition.

Method. A single-line boundary-condition change, swapping a no-flux condition for a flux-proportional one. No new field or solver machinery is introduced. # assembly_stage2.py — Robin membrane BC (replaces J[N]=0) J[N] = -k_MT * c[N-1]

Limit-check: With the default kMT = 10⁻¹⁰ m/s, the results are numerically identical to Stage 1. This further emphasizes the importance of additive building in Multiphysics models. A new term is validated against the limit in which it vanishes, confirming that membrane O₂ permeability is non-limiting under these conditions.

Result: Because kMT = 10⁻¹⁰ m/s is renders the membrane effectively impermeable under these conditions, Stage 2 produces a polarization curve numerically indistinguishable from Stage 1 (Figure 6) and is not shown separately in the comparison plots. The primary purpose of this stage is validation and confirming that the Robin membrane BC implementation is correct before introducing a physically meaningful permeability.

Stage 3: Gas-phase transport (two domains)

Mechanism: Gas-phase O₂ transport in both the GDL and the CL pores follows Fick’s law. The dissolved ionomer O₂ concentration is set by Henry’s law, a temporary simplifying assumption introduced here that defers the ionomer-film interphase resistance and is relaxed in Stage 4.

Method: Two domain meshes are concatenated into a single DOF vector, and a harmonic-mean interface diffusivity bridges the GDL/CL discontinuity. This requires a new DOF block:

u[0 : NG] = ln_c_O2_gdl # gas O2 in GDL (30 cells) u[NG : NG+NC] = ln_c_O2_cl # gas O2 in CL pores (50 cells) u[NG+NC : NG+2*NC] = phi_L u[NG+2*NC: NG+3*NC]= phi_s

Gas-phase and ionomer O₂ are coupled at every CL cell by Henry’s law: cO2,ion=Keq⋅cO2,gas. This algebraic coupling is the instantaneous-equilibrium (kv→∞) limit of the finite-rate interphase transfer introduced in Stage 4, where kv becomes finite.

At the GDL/CL interface, a harmonic-mean diffusivity handles the discontinuity between the two domains:

# gas_transport.py — interface flux with harmonic-mean diffusivity resistance = 0.5*dx_gdl/D_gdl_eff + 0.5*dx_cl/D_cl_eff J_if = -(c_cl_first - c_gdl_last) / resistance

The harmonic mean naturally emerges from requiring flux continuity across the interface and is the standard FV treatment for interfaces.16 Physically, it weights the two half-cell resistances in series: a layer with low diffusivity adjacent to the interface controls the flux, just as the smallest conductance limits current in any series-resistance circuit.

Limit-check: The effective gas-phase diffusivity is roughly 54,000× the ionomer value, so the limiting current jumps to hundreds of mA/cm², consistent with the realistic operating range of an air-fed cathode.

Result: The ohmic regime becomes visible for the first time.

Adding gas-phase transport through the GDL and CL pores is what brings the predicted current into a realistic range. Because the effective gas-phase diffusivity in the CL pores (≈6.9×10⁻⁶ m²/s, the Bruggeman-corrected value of the bulk O₂/N₂ diffusivity 3.5×10⁻⁵ m²/s) is roughly 54,000 times larger than the effective ionomer diffusivity (≈1.27×10⁻¹⁰ m²/s), Figure 10 shows the Stage 3 current reaching hundreds of mA/cm² before mass transport becomes limiting, the realistic operating range for an air-fed PEMFC cathode.

Figure 10: Stage 3 results. Stage 1 vs. Stage 3 polarization curves and O₂ concentration profiles. Gas-phase transport in the GDL and CL pores extends the achievable current density to hundreds of mA/cm², the realistic operating range for an air-fed PEMFC cathode.

Stage 4: Finite-rate interphase coupling + Maxwell–Stefan gas (the complete model)

Mechanism: Real low-Pt cathodes are limited by O₂ transport through the thin ionomer film around each Pt site, but this is ignored by the Stage 3 equilibrium assumption. Stage 4 therefore promotes the dissolved (ionomer) O₂ back to an independent field variable, couples it to the pore gas through the finite-rate interphase transfer RPT=kv(KeqcO2,gas-cO2,ion),with kv=kMT,GL⋅aGL, and moves the ORR sink onto the dissolved-O₂ equation so the kinetic expressions see the real local cO2,ion rather than an equilibrium proxy. The gas-phase flux is likewise upgraded from Fick’s law to the binary Maxwell–Stefan / stagnant-film law N=-D∇c1-xO2, for O₂ diffusing through inert, stagnant N₂.

Method: One additional DOF block for the dissolved O₂ (bringing the total DOFs to NG + 4·NC), the (1-xO2) Stefan factor in the gas flux, and initializing the Newton solver from the converged Stage 3 solution. Since Stage 4 adds physics on top of Stage 3 rather than replacing it, this method of “warm-starting” gives the solver a starting point already close to the true solution and converges far more reliably than a cold start from equilibrium. NG is nodes in gas diffusion layer. NC is nodes in catalyst layer.

This brings the total to NG + 4·NC unknowns:

u[0 : NG] = ln_c_O2_gdl # gas O2 in GDL u[NG : NG+NC] = ln_c_O2_clgas # gas O2 in CL pores u[NG+NC : NG+2*NC] = ln_c_O2_ion # dissolved (ionomer) O2 (NEW) u[NG+2*NC : NG+3*NC] = phi_L u[NG+3*NC : NG+4*NC] = phi_s

In code, the finite-rate transfer and the relocated ORR sink are assembled as:

# assembly_stage4.py — finite-rate interphase transfer R_PT = p.k_v * (p.K_eq_gas_ion * c_gas - c_ion) # net O2 rate INTO ionomer [mol/m3/s] i_ORR = R_ORR(ln_c_ion, phi_s, phi_L, p) # kinetics fed the REAL dissolved O2 R_cgas = J_cgas[:-1] - J_cgas[1:] + (-R_PT) * dxC # gas loses O2 R_cion = J_ion[:-1] - J_ion[1:] + (R_PT - i_ORR/(N_ELEC*p.F)) * dxC # ionomer gains, ORR consumes

and the Maxwell–Stefan stagnant-film correction enters the gas flux as:

# gas_transport.py — (1 - x_O2) Stefan-flow correction def _stefan_denom(c_face, c_tot): return np.maximum(1.0 - c_face / c_tot, 1e-3) # floor keeps flux well-behaved c_face = 0.5 * (c[:-1] + c[1:]) J[1:N] = -D_cl_eff * np.diff(c) / dx_cl / _stefan_denom(c_face, c_tot)

The complete Stage 4 model makes the cost of the ionomer-film resistance visible. Figure 11 overlays the Stage 1, Stage 3, and Stage 4 polarization curves, and the Stage 4 limiting current now sits about 14% below Stage 3's because the finite-rate interphase transfer throttles O₂ delivery to the reaction sites. Figure 12 highlights the gap between the dissolved-O₂ profile and its local equilibrium value, such that the interphase transport resistance is made visible.

Figure 11: Stage 4 polarization overlay. Cathode potential versus current density for Stage 1 (ionomer-phase O₂  only), Stage 3 (gas-phase transport with local Henry's law), and Stage 4 (finite-rate interphase transfer with the Maxwell-Stefan gas flux). The limiting current in Stage 4 sits below that calculated at Stage 3 (here by about 14%) because of the added ionomer-film transport resistance.

Figure 12: Stage 4 O2 concentration profiles. (a) Pore-gas O₂  through the GDL and CL at several cathode potentials, showing depletion toward the membrane. (b) Dissolved (ionomer) O₂  in the CL (solid) compared with the local equilibrium value KeqcO2,gas (dotted); the shaded gap between the two curves is the interphase transport resistance made visible.

Figure 13 decomposes the applied overpotential into its loss contributions and confirms that kinetics still dominate (about 98% at the maximum current), with the new ionomer-film mass-transport term the largest of the otherwise-small non-kinetic losses.

Figure 13: Stage 4 applied-voltage (power-loss) breakdown. The applied overpotential, UOCV - Vcathode, is decomposed into its loss contributions as a function of current density. (a) Full stacked-area decomposition into the kinetic (activation) overpotential, the ionomer-film and gas-phase mass-transport losses, and the ionic and solid ohmic losses; the dashed line marks the total UOCV - Vcathode. Kinetic overpotential dominates across the full range (about 98% at the maximum current). (b) The non-kinetic losses replotted from zero on an expanded millivolt scale, showing that the ionomer-film mass-transport resistance introduced in Stage 4 is the largest of the otherwise-small non-kinetic contributions, exceeding both the gas-phase and ohmic terms.

Limit-check: As kv→∞ the finite-rate transfer becomes instantaneous and the Stage 4 polarization curve and profiles collapse onto those calculated at Stage 3 to within 1%. Figure 14 sweeps the interphase transfer coefficient kv and recovers the Stage 3 limiting current as kv→∞, the instantaneous-equilibrium limit, confirming that Stage 3 is the fast-transfer special case of Stage 4.

Result: Implementing the finite-rate interphase transfer throttles O2 delivery to the reaction sites which pulls the limiting current below that in Stage 3. The effect of this resistance is directly visible in Figure 12 as the gap between the dissolved O2 profile and its local Henry’s law equilibrium value.

Figure 14: Limiting current versus interphase transfer rate. The Stage 4 limiting current rises with the volumetric interphase transfer coefficient kv = kMTaGL and saturates at the Stage 3 (instantaneous-equilibrium, kv→∞) value (dashed line). The star marks the default kv  = 5×104 s-1, at which the film resistance still measurably limits the current.

Limitations of Python and When to Use Specialized Software

The hand-coded Python approach developed in this chapter is deliberately minimal. This transparency into the FVM discretization and Newton iteration is its primary virtue and its primary limitation. As the physical system grows in complexity (more coupled species, stiffer kinetics, higher spatial dimensionality), the hand-coded approach hits fundamental walls that no amount of NumPy optimization can overcome. This section describes those walls and introduces the software tools purpose-built to scale past them.

Stiff Systems: Reactions with Coupled Chemical Equilibria

What is stiffness? A system of ODEs or PDEs is called stiff when it contains processes that operate on vastly different time or length scales simultaneously. Numerically, stiffness manifests as a Jacobian whose eigenvalues span many orders of magnitude. Explicit solvers and naive Newton implementations are forced to take extremely small steps to resolve the fast dynamics, making the computation either prohibitively slow or outright unstable.

The PEMFC cathode model in this paper is not particularly stiff: it contains at most five coupled fields (three in the Stage 1 core), a single kinetic expression, and Bruggeman-corrected transport coefficients that vary smoothly in space. The Newton solver converges in under 20 iterations at every voltage point.

Reactions introducing severe stiffness: In CO₂ reduction electrolyzers (CO₂R), the local chemical environment is controlled by a set of fast, reversible buffer equilibria involving CO₂, HCO₃⁻, CO₃²⁻, OH⁻, and H⁺:

| H2O | k1,k-1↔ H++OH- | K1=1×10-14 | (17) |
| --- | --- | --- | --- |
| CO2(aq)+H2O(l) | k2,k-2↔ H(aq)++HCO3(aq)- | K2=4.27×10-7 | (18) |
| HCO3- | k3,k-3↔ H++CO32- | K3=4.58×10-11 | (19) |
| CO2(aq)+OH- | k4,k-4↔ HCO3- | K4=4.27×107 | (20) |
| HCO3-+OH- | k5,k-5↔ H2O+CO32- | K5=4.58×103 | (21) |

These homogeneous reactions have rates that differ by over ten orders of magnitude, and all species are coupled through the local pH. The resulting Jacobian has eigenvalues spanning roughly twelve orders of magnitude. Several specific failure modes emerge in the hand-coded Python framework:

- Dense Jacobian scaling fails. The row-scaling approach used in solver.py normalizes each equation by its own maximum magnitude. With twelve coupled species whose concentrations range from 10-10 to 103 mol/m3, the condition number of the scaled Jacobian can still exceed 108, leading to catastrophic cancellation in scipy.linalg.solve, and correspondingly to nonconvergence or wrong answers.

- Step clamping becomes physics-specific. The fixed clamps |Δ ln c| ≤ 5 and |Δφ| ≤ 0.2 V that stabilize the fuel cell Newton iteration are insufficient for a 12-species CO₂R system. Each species needs individually tuned limits, and the optimal values depend on which regime of the polarization curve is being solved. Maintaining these limits by hand across a parameter sweep becomes error-prone.

- Sparse solvers become necessary. The 150×150 Jacobian used in the fuel cell example is small enough that scipy.linalg.solve (a dense lower-upper (LU) factorization) is fast. A 2D CO₂R model with 12 species on a 100×100 mesh produces a 120,000×120,000 Jacobian. Storing and factorizing this as a dense matrix would requires ~100 GB of memory and O(N3) operations, which is intractable on most machines. Sparse storage and iterative or sparse-direct solvers (scipy.sparse.linalg) are required, adding substantial infrastructure.

- Continuation becomes fragile. Natural voltage continuation works well for smooth polarization curves with a single solution. CO₂R systems can exhibit bistability, hysteresis, and bifurcations driven by the pH feedback between local OH⁻ production and buffer equilibria. Simple adaptive voltage stepping will miss or mistrack these features without branch-following algorithms that are not trivially implemented in NumPy.

Multidimensional Modeling: Beyond 1D

The 1D approximation is powerful precisely because it is cheap. 50 cells with 3 unknowns each can be solved in milliseconds. But it rests on an implicit assumption: that all gradients of interest are aligned with a single coordinate axis. This breaks down in several physically important situations.

Along-the-channel gradients. In a real flow-field cell, the gas composition changes as reactants are consumed along the flow channel. A 1D through-plane model at a single channel position cannot predict this variation. A 2D model (through-plane × along-channel) is needed to predict concentration gradients along the electrode, local current density distributions, and the influence of flow rate on performance.

Two-phase water management. Liquid water produced by ORR at the cathode must be expelled through the GDL pores. When water blocks pores, the effective gas diffusivity drops sharply, creating coupled two-phase flow that requires resolving saturation fields across a 2D or 3D GDL microstructure. This cannot be represented using a 1D Fickian approximation.

Irregular geometries. Bipolar plate flow channels, gas diffusion electrode microstructures, and membrane electrode assembly interfaces are inherently three-dimensional objects with complex geometries that cannot be represented on a 1D uniform mesh.

Extending the hand-coded FVM framework to 2D requires mesh connectivity tables (which cells share which faces), handling non-uniform and unstructured meshes, assembling sparse Jacobians with irregular sparsity patterns, and implementing efficient sparse linear solvers. This is not conceptually hard, but it is several thousand lines of infrastructure code that already exists in a mature, tested form in dedicated packages.

Alternative Tools for Multiphysics Modeling

These limitations motivate a family of purpose-built tools, each with different trade-offs in flexibility, performance, and ease of use.

COMSOL Multiphysics is a commercial finite-element platform with a graphical interface and a broad library of built-in physics modules covering fluid mechanics, electrochemistry, heat transfer, and structural mechanics among others. Its key strengths for electrochemical modeling are automatic mesh generation from computer-aided design (CAD) geometry, built-in stabilization for convection-dominated and stiff problems, and an interface specifically designed for coupled PDE systems. The parametric sweep, sensitivity analysis, and bifurcation tracking tools handle the difficult continuation problems that defeat naive Newton implementations. The tradeoff is cost, opacity (the solver is a black box), and limited flexibility for non-standard physics. Much of the published literature on electrochemical modeling uses COMSOL due to its accessibility for electrochemists without substantial numerical methods training.

Firedrake is an open-source Python-based finite-element framework that occupies the space between hand-coded NumPy and commercial COMSOL. It uses the Unified Form Language (UFL), a domain-specific language embedded in Python, to express the weak form of a PDE in notation that closely mirrors the mathematics.

Firedrake automatically generates optimized C code for the local element integrals, assembles the global sparse system, and invokes PETSc's suite of preconditioners and solvers. For multidimensional electrochemical problems, Firedrake provides unstructured mesh support (via Gmsh), automatic differentiation for exact Jacobians, and scalable parallel solvers, all while keeping the model specification in readable Python. Its sibling project FEniCSx offers similar capabilities with a slightly different application programming interface (API).

Julia (specifically the DifferentialEquations.jl ecosystem and Gridap.jl/Ferrite.jl for FEM) offers a compelling alternative for users who need the transparency of a hand-coded model but cannot afford the performance penalties of Python.17 Julia's just-in-time compilation produces performance competitive with Fortran and C while retaining Python-like expressiveness. DifferentialEquations.jl is widely regarded as the most sophisticated ODE/DAE/PDE solver suite available in any language, with automatic stiffness detection, implicit-explicit (IMEX) splitting, and dozens of high-order solvers with adaptive time stepping. For stiff CO₂R buffer systems, the Rodas5 or KenCarp4 solvers in Julia handle eigenvalue spreads of 1012 routinely. The ModelingToolkit.jl library can automatically derive sparse Jacobians symbolically from a high-level equation specification, eliminating the forward-difference Jacobian bottleneck entirely.

# Summary and Outlook

This chapter summarized how continuum multiphysics modeling of electrochemical systems can be accomplished using the open-source packages NumPy and SciPy. We start by introducing the physics that underpins every continuum model (conservation laws of the form ∂ϕ∂t=-∇⋅J+S) and highlighting the unique value of continuum models for electrochemists and electrochemical engineers. We also present the key concept that a collection of physical balances (e.g., mass, energy, momentum conservation) come together to form a “multiphysics” problem. Every conservation equation is coupled through its source terms or fluxes, so the equations for multiple balances must be solved together rather than one at a time. From there we assemble the numerical methodology required to solve these multiphysics problems: the discretization of a physical domain onto a finite-volume mesh, approximation of fluxes at cell faces, collection of the discrete balances into a single nonlinear residual F(u) = 0, and the solution of that system with a Newton iteration carried along the polarization curve by voltage continuation.

Once the fundamentals of both the continuum physics and the numerical methods are established, we demonstrate these techniques on a problem relevant to electrochemists—the modeling of the oxygen-reduction cathode of a polymer-electrolyte-membrane fuel cell. Rather than presenting the full model at once, we walk the reader through the process of building the multiphysics simulation in four validated stages, each switching on a single physical mechanism, introducing at most one new numerical technique, and confirming itself against a known limiting case before the next was added. This staged implementation represents a realistic development pipeline of a complex multiphysics model. The dissolved-O₂ and ohmic-charge transport are modeled first (Stage 1), followed by inclusion of a finite membrane-permeability boundary condition (Stage 2), gas-phase transport across two domains (Stage 3), and finite-rate interphase coupling with Maxwell–Stefan gas transport (Stage 4). The end product of this implementation pipeline is a complete five-field model in which every term traces back to a specific physical process, anchored by limit checks that confirm the physics of the model are consistent with electrochemical engineering intuition.

The model is used to calculate polarization curves decomposed into kinetic, ohmic, and mass-transport losses and the internal concentration and potential fields that experiments cannot directly resolve, while also providing a platform for the sensitivity and optimization studies that guide real design. Finally, we provide a detailed discussion of the limitations of Python for numerical modeling, such as its difficulty solving stiff multispecies chemistries such as CO₂ reduction, two-phase water management, and multidimensional geometries. These shortcomings motivate the need for specialized tools for numerical modeling (Julia’s DifferentialEquations.jl, Firedrake and FEniCSx, COMSOL, and OpenFOAM), each custom-built to solve numerical problems that would be intractable in NumPy and SciPy.

An important throughline of this chapter is the importance of understanding the fundamentals of both the numerical methods and the electrochemistry when developing a continuum model of a system. Knowing why a residual must be scaled, what a step clamp prevents, how a flux balance becomes a row of a matrix, and how a new mechanism is validated against a limiting case is crucial in the development and troubleshooting of continuum models of electrochemical systems. This physical and numerical intuition is applicable to the development of any electrochemical simulation, and building these skills and understanding are the key takeaways from this chapter.

References

(1)	LeVeque, R. J. Finite Difference Methods for Ordinary and Partial Differential Equations - Steady-State and Time-Dependent Problems; Society of Industrial and Applied Mathematics, 2007.

(2)	LeVeque, R. J. Finite Volume Methods for Hyperbolic Problems; 2002.

(3)	Harris, C. R.; Millman, K. J.; van der Walt, S. J.; Gommers, R.; Virtanen, P.; Cournapeau, D.; Wieser, E.; Taylor, J.; Berg, S.; Smith, N. J.; Kern, R.; Picus, M.; Hoyer, S.; van Kerkwijk, M. H.; Brett, M.; Haldane, A.; del Río, J. F.; Wiebe, M.; Peterson, P.; Gérard-Marchant, P.; Sheppard, K.; Reddy, T.; Weckesser, W.; Abbasi, H.; Gohlke, C.; Oliphant, T. E. Array Programming with NumPy. Nature. Nature Research September 17, 2020, pp 357–362. https://doi.org/10.1038/s41586-020-2649-2.

(4)	Virtanen, P.; Gommers, R.; Oliphant, T. E.; Haberland, M.; Reddy, T.; Cournapeau, D.; Burovski, E.; Peterson, P.; Weckesser, W.; Bright, J.; van der Walt, S. J.; Brett, M.; Wilson, J.; Millman, K. J.; Mayorov, N.; Nelson, A. R. J.; Jones, E.; Kern, R.; Larson, E.; Carey, C. J.; Polat, İ.; Feng, Y.; Moore, E. W.; VanderPlas, J.; Laxalde, D.; Perktold, J.; Cimrman, R.; Henriksen, I.; Quintero, E. A.; Harris, C. R.; Archibald, A. M.; Ribeiro, A. H.; Pedregosa, F.; van Mulbregt, P.; Vijaykumar, A.; Bardelli, A. Pietro; Rothberg, A.; Hilboll, A.; Kloeckner, A.; Scopatz, A.; Lee, A.; Rokem, A.; Woods, C. N.; Fulton, C.; Masson, C.; Häggström, C.; Fitzgerald, C.; Nicholson, D. A.; Hagen, D. R.; Pasechnik, D. V.; Olivetti, E.; Martin, E.; Wieser, E.; Silva, F.; Lenders, F.; Wilhelm, F.; Young, G.; Price, G. A.; Ingold, G. L.; Allen, G. E.; Lee, G. R.; Audren, H.; Probst, I.; Dietrich, J. P.; Silterra, J.; Webber, J. T.; Slavič, J.; Nothman, J.; Buchner, J.; Kulick, J.; Schönberger, J. L.; de Miranda Cardoso, J. V.; Reimer, J.; Harrington, J.; Rodríguez, J. L. C.; Nunez-Iglesias, J.; Kuczynski, J.; Tritz, K.; Thoma, M.; Newville, M.; Kümmerer, M.; Bolingbroke, M.; Tartre, M.; Pak, M.; Smith, N. J.; Nowaczyk, N.; Shebanov, N.; Pavlyk, O.; Brodtkorb, P. A.; Lee, P.; McGibbon, R. T.; Feldbauer, R.; Lewis, S.; Tygier, S.; Sievert, S.; Vigna, S.; Peterson, S.; More, S.; Pudlik, T.; Oshima, T.; Pingel, T. J.; Robitaille, T. P.; Spura, T.; Jones, T. R.; Cera, T.; Leslie, T.; Zito, T.; Krauss, T.; Upadhyay, U.; Halchenko, Y. O.; Vázquez-Baeza, Y. SciPy 1.0: Fundamental Algorithms for Scientific Computing in Python. Nat. Methods 2020, 17 (3), 261–272. https://doi.org/10.1038/s41592-019-0686-2.

(5)	Hunter, J. D. Matplotlib A 2D Graphics Environment. Comput. Sci. Eng. 2007, 9 (3), 90–95.

(6)	Newman, J.; Thomas-Alyea, K. E. Electrochemical Systems; John Wiley & Sons, Inc., 2004.

(7)	Baratta, I. A.; Dean, J. P.; Dokken, J. S.; Hale, J. S.; Richardson, C. N.; Rognes, M. E.; Scroggs, M. W.; Sime, N.; Wells, G. N. DOLFINx: The next Generation FEniCS Problem Solving Environment; 2023; Vol. 1.

(8)	Gustafsson, T.; McBain, G. Scikit-Fem: A Python Package for Finite Element Assembly. J. Open Source Softw. 2020, 5 (52), 2369. https://doi.org/10.21105/joss.02369.

(9)	Weller, H. G.; Tabor, G.; Jasak, H.; Fureby, C. A Tensorial Approach to Computational Continuum Mechanics Using Object-Oriented Techniques. Computers in Physics 1998, 12 (6), 620–631. https://doi.org/10.1063/1.168744.

(10)	Ham, D. A.; Kelly, P. H. J.; Mitchell, L.; Cotter, C. J.; Kirby, R. C.; Sagiyama, K.; Bouziani, N.; Vorderwuelbecke, S.; Gregory, T. J.; Betteridge, J.; Shapero, D. R.; Nixon-Hill, R. W.; Ward, C. J.; Farrell, P. E.; Brubeck, P. D.; Marsden, I.; Gibson, T. H.; Homolya, M.; Sun, T.; Mcrae, A. T. T.; Luporini, F.; Gregory, A.; Lange, M.; Funke, S. W.; Rathgeber, F.; Bercea, G.-T.; Markall, G. R. Firedrake User Manual; 2023.

(11)	Fuller, T. Solid-Polymer-Electrolyte Fuel Cells; 1992.

(12)	Fuller, T. F.; Harb, J. N. Electrochemical Engineering; John Wiley & Sons, Inc., 2018.

(13)	Newman, J. S.; Tobias, C. W. Theoretical Analysis of Current Distribution in Porous Electrodes You May Also like Methods for the Calculation of Polarization in Porous Electrodes Analysis of Porous Electrodes with Sparingly Soluble Reactants; 1962; Vol. 109.

(14)	Chapra, S. C.; Canale, R. P. Numerical Methods for Engineers, 6th ed.; McGraw-Hill Higher Education, 2010.

(15)	Gerhardt, M. R.; Pant, L. M.; Bui, J. C.; Crothers, A. R.; Ehlinger, V. M.; Fornaciari, J. C.; Liu, J.; Weber, A. Z. Method—Practices and Pitfalls in Voltage Breakdown Analysis of Electrochemical Energy-Conversion Systems. J. Electrochem. Soc. 2021, 168 (7), 074503. https://doi.org/10.1149/1945-7111/abf061.

(16)	Patankar, S. V. Numerical Heat Transfer and Fluid Flow; Minkowycz, W. J., Sparrow, E. M., Eds.; McGraw-Hill, 1980.

(17)	Bezanson, J.; Edelman, A.; Karpinski, S.; Shah, V. B. Julia: A Fresh Approach to Numerical Computing. SIAM Review 2017, 59 (1), 65–98. https://doi.org/10.1137/141000671.