# Introduction to Continuum Multiphysics

Real electrochemical systems are composed of discrete atoms, ions, and molecules. Yet at the length scales relevant to devices — micrometers to centimeters — individual molecular events average out into smooth, continuously varying fields: concentration, electric potential, temperature, velocity, etc. Continuum modeling exploits this by replacing the discrete molecular picture with field variables governed by partial differential equations (PDEs). Rather than tracking every ion, we track the concentration field c(x,t) and solve for how it evolves in space and time.

Continuum approaches occupy the length and time scale between molecular methods (quantum chemistry, molecular dynamics) and system-level approaches (technoeconomic models). This mesoscale is where lab-scale observables like polarization curves and concentration profiles live, making continuum models directly comparable to benchtop experiments

Throughout this chapter, we use the Oxygen Reduction Reaction (ORR) at a PEMFC cathode as a concrete worked example to illustrate each concept as it is introduced. For context, a proton exchange membrane fuel cell (PEMFC) converts the chemical energy of hydrogen directly into electricity by spatially separating oxidation and reduction: H₂ is split at the anode into protons and electrons, which travel to the cathode through the membrane and external circuit respectively. There, they converge on Pt nanoparticles dispersed within a porous catalyst layer, driving the oxygen reduction reaction (ORR) — a four-electron, four-proton reduction of O₂ to water.

This system is inherently Multiphysics in nature, simultaneously coupling O₂ diffusion through the ionomer, proton conduction through Nafion, and electron conduction through the carbon support, all linked through a nonlinear electrochemical reaction rate that depends on the local concentration and potential of every phase at once. What multiphysics is will become clearer as we continue through the following section.

# Continuum Modeling for Electrochemistry

## What can we learn?

Continuum models translate the governing physics, transport laws, kinetics, and conservation equations into quantitative predictions of measurable quantities. In electrochemistry and catalysis, the primary observables are polarization curves (current density vs. applied voltage) and faradaic efficiencies (fraction of current driving the reaction of interest). A validated continuum model that can reproduces these observables gains predictive credibility where it can then be interrogated for information that experiments cannot directly provide.

## Why is it valuable?

The central advantage of continuum modeling is access to internal state variables, like field variables, which are quantities that govern device behavior but are experimentally inaccessible. Consider what a polarization curve measurement tells you: a single number (total current) at each applied voltage. A continuum model that reproduces these experimental variables allows us to ask deeper questions in the system that aren’t accessible.

- Spatial concentration profiles: Where is O₂ being depleted? How steep is the gradient near the membrane?

- Potential distributions: how much of the applied voltage is lost to ohmic resistance in the ionomer vs. carbon support?

- Overpotential distributions: where along the catalyst layer is the reaction fastest?

- Flux decomposition: what fraction of current is carried by ionic vs. electronic pathways?

These are inaccessible to a voltmeter. They require solving the coupled PDEs on a mesh.

Another advantage is the ability to perform voltage breakdown analysis. The total applied potential can be decomposed into contributions from each loss mechanism (ohmic drop, concentration overpotential, kinetic overpotential). This decomposition is only possible after solving the full field equations, and it directly identifies the limiting process at each operating point.  This sort of information would require multiple specialized experiments to infer indirectly.

Additionally, validated models enable sensitivity analysis and optimization: systematically varying material properties (diffusivity, conductivity, catalyst loading) in a model or geometry (layer thickness) across a parameter space to identify performance guides experimental design and development.  Work that would be prohibitively expensive to determine purely experimentally.

Lastly, continuum modeling sits at a uniquely productive scale. It is coarser than molecular dynamics (no individual ion trajectories) but finer than system-level models that collapses the entire device into a lumped circuit element (membrane as a resistor, double layer as a capacitor). The mesoscale, micrometers to millimeters and microseconds to steady state, matches the operative scale of most lab electrochemical devices. Models at this scale can be validated against directly measured polarization data, then used to resolve physics in experimentally hard to reach or inaccessible domains.

# Continuum Modeling: Fundamentals and Numerical Methods

## Conservation Equations: Mass and Current

The governing equations of continuum modeling are conservation laws. For a scalar quantity ϕ (a concentration, a charge density, an energy) in a control volume, the statement is simple:

∂ϕ∂t=-∇⋅J+S

where J is the total flux of ϕ across the control volume boundaries and S is a volumetric source or sink. This single conservation equation generates the specific equations for each physical scalar quantity or field.

[ Figure]

O2+4H++4e-→2H2O

| Field, ϕ | Flux J | Source S |
| --- | --- | --- |
| Dissolved O₂ concentration cO2 | Fick's law -Deff∇c | ORR consumption via Faraday’s Law -iORR/4F (4 electrons transferred per O2 molecule) |
| Ionic current iL | Ohm's law -κeff∇ϕL | ORR generation +iORR Reaction produces ionic current as Protons are consumed at the Pt surface |
| Electronic current is | Ohm's law -σeff∇ϕs | ORR consumption -iORR Electrons drawn from Solid Phase to drive Reduction |

Here, Deff is the Bruggeman-corrected effective diffusivity of dissolved O2 in the ionomer phase, κeff is the effective ionic conductivity of the Nafion Ionomer, σeff is the effective electronic conductivity of the carbon support. Each of these transport properties are corrected from their bulk value to account for the tortuous pore network of the catalyst layer. c is the local dissolved O2 concentration, ϕL is the ionic (electrolyte) phase potential, and ϕS is the solid (electronic) phase potential. The difference (ϕS-ϕL), at any point defines the local driving force for the electrochemical reaction.

The sign of the source term reflects each phase's role in the reaction: ORR consumes O₂ and draws electrons from the solid phase (negative sources), while simultaneously generating ionic current as protons are consumed from the ionomer phase (positive source).

# What does "multiphysics" mean?

The conservation law on species concentration is a second-order PDE in 1D

∂(c(x,t))∂t-ddxDeffdcdx=S(c,ϕs,ϕL)

The source term S or iORR couples the three conservation equations together This is where the "multi" in multiphysics enters. "Multiphysics" means that multiple conservation equations are solved simultaneously because their terms (in this case, source terms) are coupled. In this fuel cell cathode model, the ORR reaction rate iORR depends on cO2, ϕs, and ϕL all at once. You cannot solve the O₂ equation in isolation as the S term requires knowing the potentials, and the charge equations require knowing the concentration. The system of three coupled PDEs must be solved simultaneously as a single nonlinear problem.

# What is a mesh?

To solve partial differential equations (PDEs) or these conservation equations numerically, the continuous spatial domain is divided into a finite number of discrete cells or elements. This grid is what we refer to as a mesh. In a 1D finite-volume mesh, the domain 0,L is partitioned into N cells of width Δx=L/N.

Each cell has a centroid, the point of its geometric centre, where the field variable (concentration, potential) is stored and treated as the cell-averaged value. This stored value is the degree of freedom, or DOF, that the solver will find.

The governing PDE is then enforced as a discrete balance on each cell, converting the continuous PDE into a finite system of algebraic equations.

Between adjacent cells sit faces: the flat interfaces that separate one cell from its neighbor. Faces are not where variables are stored , but where fluxes are computed. A flux is simply the rate at which a quantity passes through a unit area of that face per unit time. For example, Fick's law says the molar flux of O₂ across the face between cell i and cell i+1 is proportional to the concentration difference across it:

Ji+1/2=-Deffci+1-ciΔx

The subscript i+1/2 denotes the face sitting halfway between centroids i and i+1. This finite difference approximates the true derivative dc/dx. The smaller Δx is, the more accurately it captures the local gradient and the lower the discretization error. However, halving Δx doubles the number of cells N, which increases both the memory required to store the system and the computational cost of solving it. In practice, one chooses N large enough that the solution no longer changes appreciably with further refinement balancing accuracy against cost. The governing PDE is then enforced as a discrete flux balance on each cell: whatever enters through the left face minus whatever leaves through the right face must equal the net source inside the cell. For O₂ conservation with an ORR sink:

Ji-1/2-Ji+1/2net flux in+-iORR4FΔxsource=0

This is the discrete statement of ∇⋅J=S where there is algebraic equation per cell. Repeating this for all N cells convert the continuous PDE into a system of N equations in N unknowns, which can be solved by standard linear algebra or Newton iteration.

# From Mesh to Matrix

Once the domain is discretized into  N cells, the physical state of the entire system is represented as a single solution vector u, where each entry is one field variable at one cell. Enforcing the discretized conservation equation on every cell produces one algebraic equation per DOF, collected into a residual vector F(u). The solver finds the u that satisfies:

F(u)=0

For a system with nvar field variables across N cells, the vectors are arranged in species blocks — all values of variable 1 across all cells, then all values of variable 2, and so on:

u=ϕ1,1⋮ϕ1,Nϕ2,1⋮ϕ2,N⋮,      Fu=F1,1⋮F1,NF2,1⋮F2,N⋮=0

When the system is nonlinear, which is the case for any electrochemical system or model system that has Tafel or Butler-Volmer kinetics Fu=0 is solved iteratively by Newton-Raphson:

uk+1=uk-J-1Fuk

Where J=∂F/∂u is the Jacobian, a matrix of partial derivatives of every residual equation with respect to every DOF. The solver iterates, recomputing J and F at each step, until Fu falls below a tolerance. In practice, Fu will never exactly reach 0, but gets close enough that the solution is physically meaningful.

Because of how the conservation equations are constructed, each conservation equation in each cell depends only on its immediate neighbors. As a result, the Jacobian Matrix is sparse and banded. Newman exploited this matrix structure with the BAND Algorithm, drastically cutting down the amount of numerical operations necessary to find a solution. However, as we go to higher dimensions this structure is lost, the Jacobian matrix grows to an extreme size and dedicated solvers are necessary.

▶  Further reading:  Newman & Thomas-Alyea, Electrochemical Systems (4th ed., 2004) — foundational treatment of 1D porous electrode theory and the BAND algorithm.  |  LeVeque, Finite Volume Methods for Hyperbolic Problems (2002) — rigorous derivation of FV discretization and conservation properties.  |  Kelley, Solving Nonlinear Equations with Newton's Method (2003) — practical guide to Newton iteration, convergence, and step-length control.

# Types of PDEs and their Numerical Methods

The conservation equation takes on different mathematical characteristics depending on what physics it describes. PDEs are classified into three families, each requiring a different numerical treatment:

Elliptic PDES arise at steady state when the field variable is unchanging over time ∂ϕ∂t=0. A classic example in equation is Poisson’s Equation

d2Φdx2=0

The steady-state potential and concentration profiles are solutions to these elliptic PDEs. Because no time variable is present, these are solved as boundary value problems (BVPs). The solution is determined entirely by the boundary conditions. Numerically, discretization converts the elliptic PDE into a system of nonlinear algebraic equations, solved by multivariable Newton-Raphson iteration.

Parabolic PDES retain the time derivative, and describe diffusion-dominated transient processes. A classic example in the Heat Equation or Fick’s second law.

∂T(x,t)dt=κd2Tdx2,∂C(x,t)dt=Dd2Cdx2

In electrochemistry, transient models are of interest when we are looking at the dynamics of a process and are interested in seeing how a system evolves to equilibrium. Like during a sudden potential step or exploring battery charging dynamics. The standard numerical approach is the method of lines (MOL): discretize space first using FVM or FDM to convert the PDE into a large system of ODEs in time, much like we did for the elliptic PDEs, then integrating forward using wit explicit Euler or Runge-Kutta (scipy.integrate.solve_ivp in Python possesses a variety of other methods for this intergration). Parabolic problems require both boundary conditions and an initial condition.

Hyperbolic PDEs involve second-order derivatives in both time and space.

∂2ϕ∂t2-∂2ϕ∂x2=fx,t

Because this equation is second-order in both time and space, the numerical model requires the initial time derivative of the field variable in addition to the initial condition, and boundary conditions required of the Elliptic and Parabolic PDEs. Hyperbolic PDEs are uncommon in electrochemistry but has applications in Electrochemical Impedance Spectroscopy (EIS). The standard numerical approach is once again (MOL) and splitting the equation into two coupled first-order PDEs. For example, lets say that a=ϕ, and b=∂ϕ∂t

∂a∂t=b, ∂b∂t=ϕi+1-2ϕi +ϕi-1Δx2+fx,t

After spatial discretization, this becomes a system of first-order ODEs in time that can be integrated forward using the same method-of-lines approach used for parabolic problems, but now applied to a 2N dimensional state vector a,b.

# Boundary and Initial Conditions for Electrochemical Systems

As alluded to previously, the number and type of conditions required to close a PDE system depends on its classification.

For elliptic (steady-state) problems, two boundary conditions per variable are required with one at each end of the domain. No initial condition is needed because there is no time-dependence.

For parabolic (time-dependent) problems, boundary conditions are still required at each spatial boundary, but an initial condition is also needed. The full spatial profile of every field variable at t=0. In electrochemical systems the natural initial condition is the equilibrium state. For example bulk concentrations everywhere, no applied potential, no current flowing:

c(x,0)=cbulk,ϕ(x,0)=0,t=0

This represents the system before any perturbation is applied.

Even for steady-state (elliptic) problems where no formal initial condition exists, Newton-Raphson iteration requires an initial guess as the starting point for the iterative solver. The same physical intuition applies: starting from the equilibrium state (uniform concentrations, zero current, potential equal to OCV everywhere) gives the solver the best chance of converging to the physical solution. A poor initial guess can cause convergence issues. In this chapter, the initial guess is set to the exact OCV solution:

# solver.py — equilibrium initial guess ln_cO2 = np.full(N, np.log(p.c_O2_bc)) # uniform O2 at inlet value phi_L = np.zeros(N) # no ionic potential drop phi_s = np.full(N, V_cathode) # uniform solid potential = applied voltage

This is not an arbitrary choice. At open circuit with no current flowing, these are the exact solutions to all three conservation equations, making them a natural and stable starting point.

The three BC types that commonly appear in electrochemical systems are:

Dirichlet (fixed value): The variable is prescribed directly at the boundary. Examples: a large well-stirred reservoir maintains a fixed bulk concentration; a potentiostat holds the electrode at a specified potential ϕs=Vapplied.

Neumann (fixed flux): The spatial gradient of the field variable, through a flux, is prescribed. A zero-flux (insulating) Neumann BC, ∂ϕ/∂x=0, represents an impermeable wall or an electronically insulating membrane. The no-flux condition on ϕs at the CL/membrane interface in this model is one example.

Robin (mixed): The flux is coupled to the local value of the variable through an interfacial process. Consider the active catalyst surface where electrochemical reaction consumes species at a rate set by Butler–Volmer kinetics:

-Deff∂c∂x∣surface=krxn(csurface,ϕsurface)

Because the flux (left side) depends on the gradient while the reaction rate (right side) depends on the surface value, this links both the value and the derivative.

# Python Tools for Continuum Modeling

We employ a pedagogical approach to this paper using the ORR Cathode of a PEMFC and code snippets. We begin by showing the main script (run_stage1.py). All models in this repo share a common setup pattern. The driver script (run_stage1.py) imports from each module, instantiates the two core objects, and passes them to every function

# run_stage1.py — master setup from params import Params from mesh import make_mesh from kinetics import R_ORR from transport import diffusion_face_fluxes, ohmic_face_fluxes from assembly_stage1 import residual_stage1, compute_current from solver import voltage_sweep from plot_results import plot_polarization, plot_profiles p = Params() # single authoritative parameter object — passed everywhere as "p" mesh = make_mesh(p) # reads p.L_CL and p.N_CL; returns mesh.dx, mesh.xc, mesh.xf

Every function in the repo receives either p, mesh, or both as arguments. Understanding what these two objects hold is sufficient to read any subsequent code snippet.

Organizing Parameters — dataclass as a Physics Container

A recurring practical challenge in multiphysics modeling is managing the large number of physical constants, geometry parameters, and derived quantities. It is good practice to create a Python dataclass to create a self-consistent parameter container. This params.py class contains all physical constants, geometry, and material properties. Derived quantities (effective transport coefficients, boundary condition values) are computed automatically on instantiation via __post_init__ and refreshed by calling p.recompute() after any change.

# params.py @dataclass class Params: # Physical constants F: float = 96485.332 # C/mol (Faraday) R: float = 8.314463 # J/mol/K T: float = 353.15 # K (80°C) # Geometry L_CL: float = 10e-6 # m (catalyst layer thickness) N_CL: int = 50 # number of FV cells # Transport and kinetic parameters (...) # Derived quantities — auto-computed, do not set manually D_O2_eff: float = field(init=False) # Bruggeman-corrected O2 diffusivity kappa_L_eff: float = field(init=False) # effective ionic conductivity sigma_s_eff: float = field(init=False) # effective electronic conductivity (...) def __post_init__(self): self.recompute() def recompute(self): self.D_O2_eff = (self.eps_I ** self.brugg) * self.D_O2_water * self.ionomer_factor (...)

Derived quantities like the Bruggeman-corrected effective diffusivity Deff=εI1.5⋅Dbulk are recomputed automatically when Params is instantiated, and can be refreshed after any parameter change by calling p.recompute().

Building a Mesh in NumPy

The first step in any continuum simulation is constructing the mesh. For a 1D cell-centered finite-volume mesh, this is straightforward: divide the domain 0,L into N uniform cells of width Δx = L/N, place a degree of freedom at each cell centroid, and track the face positions where fluxes are evaluated.

In this repo, the mesh is encapsulated in a simple Python dataclass mesh.py. It holds the geometry of the 1D grid. Built from p via make_mesh(p), which reads p.L_CL and p.N_CL. All flux calculations downstream use mesh.dx as the cell width.

# mesh.py @dataclass class CLMesh: L: float # domain length [m] N: int # number of cells def __post_init__(self): self.dx = self.L / self.N # cell width self.xc = (np.arange(self.N) + 0.5) * self.dx # cell centres (DOF locations) self.xf = np.arange(self.N + 1) * self.dx # face positions (flux locations) def make_mesh(p) -> CLMesh: return CLMesh(L=p.L_CL, N=p.N_CL) def make_gdl_mesh(p) -> CLMesh: return CLMesh(L=p.L_GDL, N=p.N_GDL)

xc holds the N cell-centre coordinates where field variables (cO2, ϕL, ϕs) are stored. xf holds the N+1 face positions where fluxes are computed. The offset of 0.5 in the centroid formula is the defining feature of a cell-centered scheme. The first cell center sits half a cell-width in from the boundary, not on it.

For multi-domain problems (Stage 3), separate meshes are created for the GDL and CL and their DOF vectors concatenated, which is discussed further in Section 4.

cl_mesh = make_mesh(p) # 50 cells over L_CL = 10 μm gdl_mesh = make_gdl_mesh(p) # 30 cells over L_GDL = 200 μm

No external meshing library is needed for 1D uniform grids. For 2D/3D or unstructured problems, dedicated tools like Gmsh (via the gmsh Python API) or meshio are commonly used.

The remaining modules each encapsulate one layer of the numerical stack:

| Module | Contents / key functions |
| --- | --- |
| kinetics.py | R_ORR(): volumetric ORR rate [A/m³] via Tafel kinetics; np.clip guards against overflow at large overpotentials. |
| transport.py | diffusion_face_fluxes(), ohmic_face_fluxes(): face-centred Fick and Ohm flux arrays (shape N+1) with ghost-cell BC treatment. |
| assembly_stage1.py | residual_stage1(): assembles the 3N residual vector; compute_current() and current_from_flux() for the three-way consistency check. |
| solver.py | newton_solve(): damped Newton with row scaling and per-DOF step clamping. voltage_sweep(): adaptive natural continuation from OCV to high overpotential. |
| cache.py | save_cache() / load_cache(): .npz cache for sweep results with stored Params JSON for stale-cache detection on reload. |
| plot_results.py | plot_polarization(), plot_profiles(), plot_ir_breakdown(), plot_consistency_check(): all diagnostic figures for Stage 1. |

| File | Content |
| --- | --- |
| kinetics.py | R_ORR(ln_cO2, phi_s, phi_L, p) volumetric ORR rate [A/m³] |
| transport.py | diffusion_face_fluxes(...), ohmic_face_fluxes(...) Fick/Ohm fluxes at cell faces |
| assembly_stage1.py | residual_stage1(u, mesh, p, V) full FV residual vector F(u) |
| solver.py | newton_solve(...), voltage_sweep(...) Newton iteration + continuation |
| cache.py | save_cache(...), load_cache(...) binary cache for voltage sweep results |
| plot_results.py | plot_polarization(...), plot_profiles(...), etc. diagnostic figures |

The Newman BAND Method — The Canonical Electrochemical Approach

Before modern scientific Python, the standard tool for 1D electrochemical continuum models was the BAND algorithm, introduced by John Newman in the 1960s and documented in Electrochemical Systems (Newman & Thomas-Alyea). BAND exploits the block-tridiagonal structure of the Jacobian that arises from a 1D finite-difference or finite-volume discretization: because each cell only couples to its two immediate neighbors, the 3N ×3N system has a banded structure that can be solved in ON operations by Gaussian elimination rather than ON3. Newman's FORTRAN implementation of BAND became the de facto standard for decades of electrochemical simulation. Many published models in the fuel cell and battery literature were written directly in the BAND framework. Understanding BAND is still valuable because it makes the sparsity structure of the discretized equations explicit, but today's scientific Python stack provides more flexible alternatives with comparable performance for the problem sizes typical in 1D modeling.

FDM, FVM, and FEM — What SciPy Actually Provides

A common source of confusion for newcomers: SciPy does not contain FVM or FEM solvers. What it provides is the linear algebra backbone — scipy.linalg.solve, sparse solvers in scipy.sparse.linalg — that any discretization method ultimately requires. The discretization itself (converting PDEs into matrix equations) must be implemented by the user or by a specialized package.

The three main discretization families, and how they appear in practice:

| Method | Core idea | When to use | Python tools |
| --- | --- | --- | --- |
| FDM (finite difference) | Replace derivatives with difference quotients on a structured grid | Simple geometries, quick prototyping | NumPy (by hand), findiff |
| FVM (finite volume) | Integrate conservation law over each cell; fluxes are the primary unknowns | Conservation laws (mass, charge, momentum) — natural fit for electrochemistry | NumPy (by hand), fipy |
| FEM (finite element) | Multiply PDE by test functions; minimize weighted residual | Complex geometries, structural mechanics, general PDEs | FEniCSx, scikit-fem, dolfinx |

This repo uses FVM implemented entirely in NumPy. The face-flux functions in transport.py are the core of the discretization:

# transport.py — Fick's law face fluxes (FVM) def diffusion_face_fluxes(ln_cO2, dx, D_eff, bc_left, bc_right): c = np.exp(ln_cO2) # recover concentration from log-stored DOF J = np.empty(len(c) + 1) J[0] = -D_eff * (c[0] - bc_left) / (0.5 * dx) # left Dirichlet BC J[1:-1] = -D_eff * np.diff(c) / dx # interior faces J[-1] = 0.0 # right no-flux BC return J # shape (N+1,)

Called in the assembly as:

# assembly_stage1.py J_O2 = diffusion_face_fluxes(ln_cO2, mesh.dx, p.D_O2_eff, bc_left=p.c_O2_bc, bc_right=None)

The FV residual for the O₂ conservation equation at cell i is then simply:

R_O2 = J_O2[:-1] - J_O2[1:] + (-i_ORR / (4*F)) * dx

This is the discrete statement of ∇⋅J=S: flux in minus flux out plus source equals zero.

Specialized Packages

For problems beyond 1D uniform grids, several Python packages provide higher-level FVM/FEM infrastructure:

| Package | Method | Strengths | Typical use case |
| --- | --- | --- | --- |
| fipy | FVM | Designed for diffusion-reaction-convection PDEs; syntax close to the continuous equations | Multi-component transport, phase-field models |
| FEniCSx / dolfinx | FEM | Full PDE solver framework; handles 2D/3D unstructured meshes; automatic differentiation for Jacobian | Complex geometries, fluid mechanics |
| scikit-fem | FEM | Lightweight, NumPy-native; easy to customize | Teaching, small 2D/3D problems |
| OpenFOAM (via fluidfoam) | FVM | Industrial-grade CFD; handles turbulence, two-phase flow | Full 3D reactor modeling |
| COMSOL (via LiveLink) | FEM | Commercial; GUI-driven; broad physics library | Rapid prototyping, complex multiphysics |

The deliberate choice in this repo to avoid all of these — implementing the FVM from scratch in ~300 lines of NumPy — is itself pedagogically motivated. Every operation is visible and traceable: the mesh, the flux formula, the residual, the Jacobian.

# Example PEMFC Cathode ORR Model

Physical System and Model Schematic

(Figure placeholder — model schematic to be inserted here)

The system is the cathode side of a proton-exchange membrane fuel cell (PEMFC) operating at 80°C. Molecular oxygen diffuses from a gas channel through the gas diffusion layer (GDL) and into the catalyst layer (CL), where it is reduced electrochemically at Pt nanoparticles dispersed on a carbon support and coated with Nafion ionomer. The overall half-reaction is:

O2+4H++4e-→2H2O  U∘=1.229 V vs SHE

The 1D spatial coordinate x runs from the GDL/CL interface (x=0, gas inlet) to the CL/membrane interface (x=LCL=10 μm). Three distinct phases coexist within the CL, each with its own volume fraction and transport properties:

| Phase | Volume fraction | Transport | Field variable |
| --- | --- | --- | --- |
| Ionomer (Nafion) | εI=0.43 | Dissolved O₂, ionic current | cO2, ϕL |
| Solid (Pt/C) | εS=0.23 | Electronic current | ϕs |
| Gas pores | εG=0.34 | Gas-phase O₂ (Stage 3) | cO2gas |

Transport in porous media is described by Bruggeman correction: the effective diffusivity or conductivity of a phase is scaled by its volume fraction raised to the Bruggeman exponent (typically 1.5), Deff=ε1.5Dbulk, which accounts for the increased path length through the tortuous pore network.

Governing Equations

Three coupled conservation equations govern the steady-state system:

ddxDeffiondcO2dx=iORR4F

ddxκeffdϕLdx=-iORR

ddxσeffdϕsdx=+iORR

The signs are set by the ORR source/sink structure: the reaction consumes O₂ and ionic current while producing electronic current in the solid phase. The coupling term — present in all three equations — is the volumetric ORR current density iORRAm3of Cl, given by Tafel kinetics:

iORR=av,i0cO2crefγexp!-αcFηRT

where η=(ϕs-ϕL)-Ueq(cO2) is the local kinetic overpotential and av=2×107 m-1 is the volumetric Pt surface area. This is the Tafel limit of Butler-Volmer kinetics, valid when the cathodic overpotential is large enough that the anodic term is negligible. In code:

# kinetics.py def R_ORR(ln_cO2, phi_s, phi_L, p): c_O2 = np.exp(ln_cO2) U_eq = p.U_ORR_eq(c_O2) # Nernst-corrected equilibrium eta = (phi_s - phi_L) - U_eq # local overpotential conc = np.maximum(c_O2 / p.c_O2_ref, 0.0) ** p.gamma arg = np.clip(-p.alpha * p.F * eta / (p.R*p.T), -700., 700.) return p.a_v * p.i0 * conc * np.exp(arg) # [A/m³_CL]

The argument clamp (np.clip) prevents IEEE floating-point overflow at large overpotentials — a practical necessity when sweeping from OCV to 0.35 V.

Boundary Conditions

Each second-order ODE requires two boundary conditions. The full set is:

| Variable | x=0 (GDL/CL) | x=LCL (CL/membrane) |
| --- | --- | --- |
| cO2 | Dirichlet: c=cbc (Henry's law equilibrium) | No-flux: dc/dx=0 (Stage 1/2) |
| ϕL | No-flux: dϕL/dx=0 (no ionic current crosses into GDL) | Dirichlet: ϕL=0 V (membrane reference) |
| ϕs | Dirichlet: ϕs=Vcathode (applied potential) | No-flux: dϕs/dx=0 (no electron current into membrane) |

Dirichlet at x=0, ϕs=Vcathode: The carbon support is connected to the external circuit. The applied cathode potential is held fixed here — this is the control variable swept during simulation.

No-flux (Neumann) at x=LCL, ϕs: No electronic current flows into the proton-conducting membrane. The solid-phase current has been entirely converted to ionic current by the time it reaches the membrane.

Dirichlet at x=LCL, ϕL=0: The membrane surface is taken as the electrostatic reference potential for the ionic phase. All ionic potential values inside the CL are measured relative to this.

No-flux at x=0, ϕL: Ionic current (protons) cannot cross into the electronically conducting GDL — it enters the CL from the membrane side only.

These BCs have a clean physical interpretation: solid current enters from the GDL side and is gradually converted to ionic current across the CL by the distributed ORR reaction. At the GDL face, all current is electronic; at the membrane face, all current is ionic. This current handoff is a fundamental feature of porous electrode theory, first formalized by Newman and Tobias (1962).

Multiphysics Coupling and the DOF Vector

The three equations cannot be solved independently because iORR depends on all three unknowns simultaneously. We therefore assemble them into a single nonlinear system F(u)=0 where the DOF vector u concatenates all field values across all cells:

u[0·N : 1·N] = ln(c_O2) # log-stored dissolved O2 u[1·N : 2·N] = phi_L # ionic potential u[2·N : 3·N] = phi_s # solid potential

The O₂ concentration is log-stored — we solve for ln(cO2) rather than cO2 itself. This choice has two benefits: it enforces positivity automatically (since c=eu>0 for any finite u), and it equalizes the magnitude of DOFs that otherwise span many orders of magnitude (cO2∼10-4 mol/m3 vs ϕ∼1 V).

The FV residual for each cell assembles the three conservation equations. For cell i:

# assembly_stage1.py — FV residual assembly i_ORR = R_ORR(ln_cO2, phi_s, phi_L, p) # coupling term J_O2 = diffusion_face_fluxes(ln_cO2, dx, p.D_O2_eff, bc_left=p.c_O2_bc, bc_right=None) i_L = ohmic_face_fluxes(phi_L, dx, p.kappa_L_eff, bc_left=None, bc_right=p.phi_L_mem) i_s = ohmic_face_fluxes(phi_s, dx, p.sigma_s_eff, bc_left=V_cathode, bc_right=None) R_O2 = J_O2[:-1] - J_O2[1:] + (-i_ORR / (N_ELEC * p.F)) * dx R_phiL = i_L[:-1] - i_L[1:] + (+i_ORR) * dx R_phiS = i_s[:-1] - i_s[1:] + (-i_ORR) * dx

Each residual has the form Fi=Ji-1/2-Ji+1/2+SiΔx: net flux out of the cell plus source equals zero.

Newton Solver and Voltage Continuation

The assembled system F(u)=0 is nonlinear due to the exponential in iORR. We solve it with a damped Newton method: given a current iterate u(k), compute the Jacobian J=∂F/∂u, then update:

u(k+1)=u(k)-J-1F(u(k))

The Jacobian is computed by forward finite differences — perturbing each DOF by h=ε⋅max(|uj|,1) with ε=10-7 and measuring the change in residual. This requires NDOF residual evaluations per Newton step but requires no analytical derivatives.

Two practical issues arise in practice:

Row scaling. The O₂ residuals (scale ∼10-9) and potential residuals (scale ∼108) differ by many orders of magnitude. Without scaling, the linear solver treats the small residuals as negligible. The fix is to normalize each equation by its own infinity norm before solving:

scale = np.maximum(np.abs(J).max(axis=1), 1e-30) J_sc = J / scale[:, None] F_sc = F0 / scale du = scipy.linalg.solve(J_sc, -F_sc)

Step clamping. Near the starting guess, Newton steps can be large enough to push the iterate into unphysical territory (negative concentrations, potential jumps of volts). Per-DOF step limits prevent this:

# solver.py du[0*N:1*N] = np.clip(du[0*N:1*N], -5.0, 5.0) # max 5 log-units in ln(c_O2) du[1*N:2*N] = np.clip(du[1*N:2*N], -0.2, 0.2) # max 200 mV in phi_L du[2*N:3*N] = np.clip(du[2*N:3*N], -0.2, 0.2) # max 200 mV in phi_s

To generate a full polarization curve, we use natural continuation: start at a voltage close to OCV (1.00 V) where the solution is nearly trivial, solve Newton to convergence, then use that converged solution as the initial guess for the next voltage step. The step size adapts based on convergence difficulty:

# solver.py — adaptive step size logic if n_iter <= 5: dV = min(dV * 1.5, dV_max) # easy convergence: grow step elif n_iter > 20: dV = max(dV * 0.5, dV_min) # slow convergence: shrink step # if Newton fails entirely: halve dV and retry same voltage

This produces a dense, automatically resolved polarization curve from OCV down to the mass-transport-limited regime at 0.35 V with no user tuning.

Staged Model Progression

Rather than implementing the full model at once, the code builds complexity in three validated stages. Each stage adds one physical mechanism and can be verified independently before introducing the next.

Stage 1 solves the minimal system: ionomer-phase O₂ diffusion, Tafel ORR kinetics, and ohmic charge conservation in the CL only. The O₂ boundary condition at the membrane is zero-flux (the membrane is impermeable). This is the simplest physically complete model that can produce a polarization curve.

Stage 2 modifies the membrane-side O₂ boundary condition to a Sherwood-Reynolds mass-transfer condition, JO2|x=L=-kMTcO2, parameterizing finite O₂ permeability of the membrane. With the default kMT=10-10 m/s, results are numerically identical to Stage 1 — confirming that membrane O₂ permeability is not limiting under these conditions. Stage 2 is primarily a verification step.

Stage 3 adds gas-phase O₂ transport in both GDL and CL pores. This requires a new DOF block:

u[0 : NG] = ln_c_O2_gdl # gas O2 in GDL (30 cells) u[NG : NG+NC] = ln_c_O2_cl # gas O2 in CL pores (50 cells) u[NG+NC : NG+2*NC] = phi_L u[NG+2*NC: NG+3*NC]= phi_s

Gas-phase and ionomer O₂ are coupled at every CL cell by Henry's law equilibrium: cO2ion=Keq⋅cO2gas. At the GDL/CL interface, a harmonic-mean diffusivity handles the discontinuity between the two domains:

# gas_transport.py — interface flux with harmonic-mean diffusivity resistance = 0.5*dx_gdl/D_gdl_eff + 0.5*dx_cl/D_cl_eff J_if = -(c_cl_first - c_gdl_last) / resistance

The harmonic mean naturally emerges from requiring flux continuity across the interface and is the standard FV treatment for heterogeneous media. Physically, it weights the two half-cell resistances in series: a layer with low diffusivity adjacent to the interface controls the flux, just as the smallest conductance limits current in any series-resistance circuit.

Verification: Three-Way Current Consistency

A converged numerical solution should satisfy global conservation exactly. The total current density can be computed three independent ways:

- Volumetric integral: J=0LCLiORR,dx≈i​iORR,iΔx

- Solid flux at inlet: J=-σeffdϕsdxx=0

- Ionic flux at membrane: J=-κeffdϕLdxx=L

All three must agree to better than 1% for a physically valid solution. This is automated in run_stage1.py and serves as a built-in sanity check after every sweep:

Physicality checks:

phi_s monotone decreasing     : PASS

phi_L >= 0 throughout         : PASS

c_O2 depletes inlet->membrane : PASS

3-way i_total agreement       : PASS  (err_solid=0.012%, err_ionic=0.008%)

Near-OCV current < 1 mA/cm2  : PASS  (J=0.0003 mA/cm2)

Results

Figure: Stage 1 polarization curve (V_cathode vs. J). The steep near-OCV drop is governed by Tafel kinetics; the mild mass-transport tail reflects ionomer-phase O2 depletion near the membrane.

Figure: Stage 1 spatial profiles at four sampled voltages. (a) O2 concentration; (b) ionic potential φ_L (mV); (c) solid potential φ_s; (d) kinetic overpotential η = (φ_s − φ_L) − U_eq (mV). Dark curves correspond to higher overpotential; at high overpotential the reaction is concentrated near the GDL/CL interface (x = 0).

Figure: Voltage-loss breakdown for Stage 1. Left: stacked-area decomposition showing kinetic overpotential accounting for >99% of total loss. Right: ohmic losses on an expanded millivolt scale; ionic (Nafion) resistance exceeds solid (carbon) resistance by ~27×, though both are negligible at Stage 1 currents.

Figure: Face-centred flux profiles at sampled voltages. Left: O2 diffusion flux J_O2(x) entering at the GDL face (x = 0) and falling to zero at the no-flux membrane face (x = L_CL). Right: solid-phase i_s (solid lines) and ionic-phase i_L (dashed lines) current densities; i_s decreases and i_L grows across the CL so their sum equals i_total everywhere, illustrating charge conservation.

Figure: Stage 3 results. Stage 1 vs. Stage 3 polarization curves and O2 concentration profiles. Gas-phase transport in the GDL and CL pores extends the achievable current density to hundreds of mA/cm², the realistic operating range for an air-fed PEMFC cathode.

The Stage 1 polarization curve shows two characteristic regimes: a steep near-OCV drop dominated by Tafel kinetics, and a gradual mass-transport tail at higher current densities as O₂ is depleted near the membrane. Stage 1 currents do not exceed ~3 mA/cm², at which ionic and solid-phase ohmic losses combined remain below 0.1% of the total overpotential — there is no discernible ohmic regime at this scale, consistent with the IR breakdown figure. Spatial profiles at several voltages reveal that at high overpotential the reaction is localized near the GDL/CL interface — O₂ is consumed before it can penetrate deep into the CL.

The IR breakdown figure is two-panel: the left panel shows the full voltage loss decomposition (kinetic overpotential dominates at >99% for Stage 1 currents, consistent with a kinetically limited regime), and the right panel zooms into the ohmic contributions on an expanded millivolt scale. The ionic (Nafion) resistance exceeds the solid (carbon) resistance by roughly 27× — a direct consequence of the Bruggeman-corrected conductivities computed in Params.recompute() — but both are negligible at Stage 1 current densities. Ohmic losses become significant at the higher currents reached in Stage 3.

Stage 3 introduces gas-phase transport in the GDL and CL pores. Because the effective gas-phase diffusivity in the CL pores (DO2/N2∼3.5×10-5 m2/s≈6.9×10⁻⁶ m²/s) is approximately 54,000 times larger than the effective ionomer diffusivity (≈1.27×10⁻¹⁰ m²/s), Stage 3 currents reach hundreds of mA/cm² before mass-transport limitation becomes apparent — a realistic prediction for an air-fed PEMFC cathode.

# Limitations of Python and Motivation for Specialized Software

The hand-coded Python approach developed in this paper is deliberately minimal: roughly 500 lines of NumPy that expose every step of the FVM discretization and Newton iteration. This transparency is its primary virtue and its primary limitation. As the physical system grows in complexity — more coupled species, stiffer kinetics, higher spatial dimensionality — the hand-coded approach hits fundamental walls that no amount of NumPy optimization can overcome. This section describes those walls and introduces the software tools purpose-built to scale past them.

Stiff Systems: CO₂ Reduction with Chemical Buffers

What is stiffness? A system of ODEs or PDEs is called stiff when it contains processes that operate on vastly different time or length scales simultaneously. Numerically, stiffness manifests as a Jacobian whose eigenvalues span many orders of magnitude. Explicit solvers and naive Newton implementations are forced to take extremely small steps to resolve the fast dynamics, making the computation either prohibitively slow or outright unstable.

The PEMFC cathode model in this paper is not particularly stiff: it contains three coupled fields, a single kinetic expression, and Bruggeman-corrected transport coefficients that vary smoothly in space. The Newton solver converges in under 20 iterations at every voltage point.

CO₂ reduction introduces severe stiffness. In CO₂ reduction electrolyzers (CO₂R), the local chemical environment is controlled by a set of fast, reversible buffer equilibria involving CO₂, HCO₃⁻, CO₃²⁻, OH⁻, and H⁺:

CO2+OH-⇌HCO3-  kf∼104 M-1s-1

These homogeneous reactions are orders of magnitude faster than the electrochemical reactions they accompany, and they couple every species to every other species through the local pH. The resulting Jacobian has eigenvalues spanning roughly twelve orders of magnitude. Several specific failure modes emerge in the hand-coded Python framework:

Dense Jacobian scaling fails. The row-scaling approach used in solver.py normalizes each equation by its own maximum magnitude. With twelve coupled species whose concentrations range from 10-10 to 103 mol/m3, the condition number of the scaled Jacobian can still exceed 108, leading to catastrophic cancellation in scipy.linalg.solve and nonconvergence or wrong answers.

Step clamping becomes physics-specific. The fixed clamps |Δ ln c| ≤ 5 and |Δ\phi| ≤ 0.2~\text{V} that stabilize the fuel cell Newton iteration are insufficient for a 12-species CO₂R system. Each species needs individually tuned limits, and the optimal values depend on which regime of the polarization curve is being solved. Maintaining this by hand across a parameter sweep becomes error-prone.

Sparse solvers become necessary. The fuel cell model's 150×150 Jacobian is small enough that scipy.linalg.solve (a dense LU factorization) is fast. A 2D CO₂R model with 12 species on a 100×100 mesh produces a 120,000×120,000 Jacobian. Storing and factorizing this as a dense matrix requires ~115 GB of memory and O(N3) operations — completely intractable. Sparse storage and iterative or sparse-direct solvers (scipy.sparse.linalg) are required, adding substantial infrastructure.

Continuation becomes fragile. Natural voltage continuation works well for smooth polarization curves with a single solution branch. CO₂R systems can exhibit bistability, hysteresis, and bifurcations driven by the pH feedback between local OH⁻ production and buffer equilibria. Simple adaptive stepping will miss or mistrack these features without branch-following algorithms that are not trivially implemented in NumPy.

Multidimensional Modeling: Beyond 1D

The 1D approximation is powerful precisely because it is cheap: 50 cells, 3 unknowns each, solved in milliseconds. But it rests on an implicit assumption — that all gradients of interest are aligned with a single coordinate axis. This breaks down in several physically important situations.

Along-the-channel gradients. In a real flow-field cell, the gas composition changes as reactants are consumed along the flow channel. A 1D through-plane model at a single channel position cannot predict this variation. A 2D model (through-plane × along-channel) is needed to predict concentration gradients along the electrode, local current density distributions, and the influence of flow rate on performance.

Two-phase water management. Liquid water produced by ORR at the cathode must be expelled through the GDL pores. When water blocks pores, the effective gas diffusivity drops sharply, creating coupled two-phase flow that requires resolving saturation fields across a 2D or 3D GDL microstructure. This is entirely beyond the 1D Fickian approximation.

Irregular geometries. Bipolar plate flow channels, gas diffusion electrode microstructures, and membrane electrode assembly interfaces are inherently three-dimensional objects with complex geometries that cannot be represented on a 1D uniform mesh.

Extending the hand-coded FVM framework to 2D requires building mesh connectivity tables (which cells share which faces), handling non-uniform and unstructured meshes, assembling sparse Jacobians with irregular sparsity patterns, and implementing efficient sparse linear solvers. This is not conceptually hard — but it is several thousand lines of infrastructure code that already exists in mature, tested form in dedicated packages. Reinventing it from scratch is rarely the right choice.

Alternative Tools

These limitations motivate a family of purpose-built tools, each with different trade-offs in flexibility, performance, and ease of use.

COMSOL Multiphysics is a commercial finite-element platform with a graphical interface and a broad library of built-in physics modules covering fluid mechanics, electrochemistry, heat transfer, and structural mechanics. Its key strengths for electrochemical modeling are: automatic mesh generation from CAD geometry, built-in stabilization for convection-dominated and stiff problems, and an interface specifically designed for coupled PDE systems. The parametric sweep, sensitivity analysis, and bifurcation tracking tools handle the difficult continuation problems that defeat naive Newton implementations. The tradeoff is cost, opacity (the solver is a black box), and limited flexibility for non-standard physics. Much of the published literature on BPM, CO₂ electrolyzer, and PEMFC modeling uses COMSOL, including the work from which the PhD thesis chapter in this paper is drawn.

Julia (specifically the DifferentialEquations.jl ecosystem and Gridap.jl/Ferrite.jl for FEM) offers a compelling alternative for users who need the transparency of a hand-coded model but cannot afford the performance penalties of Python. Julia's just-in-time compilation produces performance competitive with Fortran and C while retaining Python-like expressiveness. DifferentialEquations.jl is widely regarded as the most sophisticated ODE/DAE/PDE solver suite available in any language, with automatic stiffness detection, implicit-explicit (IMEX) splitting, and dozens of high-order solvers with adaptive time stepping. For stiff CO₂R buffer systems, the Rodas5 or KenCarp4 solvers in Julia handle eigenvalue spreads of 1012 routinely. The ModelingToolkit.jl library can automatically derive sparse Jacobians symbolically from a high-level equation specification — eliminating the forward-difference Jacobian bottleneck entirely.

Firedrake is an open-source Python-based finite-element framework that occupies the space between hand-coded NumPy and commercial COMSOL. It uses the Unified Form Language (UFL) — a domain-specific language embedded in Python — to express the weak form of a PDE in notation that closely mirrors the mathematics:

# Firedrake: Poisson equation in UFL notation

F = inner(grad(u), grad(v)) * dx - f*v*dx

solve(F == 0, u, bcs)

Firedrake automatically generates optimized C code for the local element integrals, assembles the global sparse system, and invokes PETSc's suite of preconditioners and solvers. For multidimensional electrochemical problems, Firedrake provides unstructured mesh support (via Gmsh), automatic differentiation for exact Jacobians, and scalable parallel solvers — all while keeping the model specification in readable Python. Its sibling project FEniCSx offers similar capabilities with a slightly different API.

Choosing the Right Tool

No single tool dominates across all use cases. A practical guide:

| Scenario | Recommended tool |
| --- | --- |
| 1D model, pedagogical clarity, full code control | NumPy/SciPy (this paper) |
| 1D–2D model, stiff multispecies kinetics, high performance | Julia + DifferentialEquations.jl |
| 2D–3D model, complex geometry, open-source, parallel | Firedrake or FEniCSx |
| 2D–3D model, rapid prototyping, commercial support | COMSOL |
| Production-scale 3D CFD with electrochemistry | OpenFOAM + custom solvers |

The hand-coded Python model in this paper remains the right starting point: it builds the physical and numerical intuition needed to use any of the above tools intelligently. Understanding why row scaling is necessary, what step clamping prevents, and how a residual is assembled makes the difference between a user who can diagnose solver failures and one who cannot.